7.9 SECURITY IN AD HOC WIRELESS NETWORKS

As mentioned earlier, due to the unique characteristics of ad hoc wireless networks, such networks are highly vulnerable to security attacks compared to wired networks or infrastructure-based wireless networks. The following sections discuss the various security requirements in ad hoc wireless networks, the different types of attacks possible in such networks, and some of the solutions proposed for ensuring network security.

7.10 NETWORK SECURITY REQUIREMENTS

A security protocol for ad hoc wireless networks should satisfy the following requirements. The requirements listed below should in fact be met by security protocols for other types of networks also.

- Confidentiality: The data sent by the sender (source node) must be comprehensible only to the intended receiver (destination node). Though an intruder might get hold of the data being sent, he/she must not be able to derive any useful information out of the data. One of the popular techniques used for ensuring confidentiality is data encryption.

- Integrity: The data sent by the source node should reach the destination node as it was sent: unaltered. In other words, it should not be possible for any malicious node in the network to tamper with the data during transmission.

• Availability: The network should remain operational all the time. It must be robust enough to tolerate link failures and also be capable of surviving various attacks mounted on it. It should be able to provide the guaranteed services whenever an authorized user requires them.

• Non-repudiation: Non-repudiation is a mechanism to guarantee that the sender of a message cannot later deny having sent the message and that the recipient cannot deny having received the message. Digital signatures, which function as unique identifiers for each user, much like a written signature, are used commonly for this purpose.

7.11 ISSUES AND CHALLENGES IN SECURITY PROVISIONING

Designing a foolproof security protocol for ad hoc wireless is a very challenging task. This is mainly because of certain unique characteristics of ad hoc wireless networks, namely, shared broadcast radio channel, insecure operating environment, lack of central authority, lack of association among nodes, limited availability of resources, and physical vulnerability. A detailed discussion on how each of the above mentioned characteristics causes difficulty in providing security in ad hoc wireless networks is given below.

• Shared broadcast radio channel: Unlike in wired networks where a separate dedicated transmission line can be provided between a pair of end users, the radio channel used for communication in ad hoc wireless networks is broadcast in nature and is shared by all nodes in the network. Data transmitted by a node is received by all nodes within its direct transmission range. So a malicious node could easily obtain data being transmitted in the network. This problem can be minimized to a certain extent by using directional antennas.

• Insecure operational environment: The operating environments where ad hoc wireless networks are used may not always be secure. One important application of such networks is in battlefields. In such applications, nodes may move in and out of hostile and insecure enemy territory, where they would be highly vulnerable to security attacks.

• Lack of central authority: In wired networks and infrastructure-based wireless networks, it would be possible to monitor the traffic on the network through certain important central points (such as routers, base stations, and access points) and implement security mechanisms at such points. Since ad hoc wireless networks do not have any such central points, these mechanisms cannot be applied in ad hoc wireless networks.

• Lack of association: Since these networks are dynamic in nature, a node can join or leave the network at any point of the time. If no proper authentication mechanism is used for associating nodes with a network, an intruder would be able to join into the network quite easily and carry out his/her attacks.

- Limited resource availability: Resources such as bandwidth, battery power, and computational power (to a certain extent) are scarce in ad hoc wireless networks. Hence, it is difficult to implement complex cryptography-based security mechanisms in such networks.

- Physical vulnerability: Nodes in these networks are usually compact and hand-held in nature. They could get damaged easily and are also vulnerable to theft.

7.12 NETWORK SECURITY ATTACKS

Attacks on ad hoc wireless networks can be classified into two broad categories, namely, passive and active attacks. A passive attack does not disrupt the operation of the network; the adversary snoops the data exchanged in the network without altering it. Here, the requirement of confidentiality can be violated if an adversary is also able to interpret the data gathered through snooping. Detection of passive attacks is very difficult since the operation of the network itself does not get affected. One way of overcoming such problems is to use powerful encryption mechanisms to encrypt the data being transmitted, thereby making it impossible for eavesdroppers to obtain any useful information from the data overheard.

An active attack attempts to alter or destroy the data being exchanged in the network, thereby disrupting the normal functioning of the network. Active attacks can be classified further into two categories, namely, external and internal attacks. External attacks are carried out by nodes that do not belong to the network. These attacks can be prevented by using standard security mechanisms such as encryption techniques and firewalls.7 Internal attacks are from compromised nodes that are actually part of the network. Since the adversaries are already part of the network as authorized nodes, internal attacks are more severe and difficult to detect when compared to external attacks.

7 A firewall is used to separate a local network from the outside world. It is a software which works closely with a router program and filters all packets entering the network to determine whether or not to forward those packets toward their intended destinations. A firewall protects the resources of a private network from malicious intruders on foreign networks such as the Internet. In an ad hoc wireless network, the firewall software could be installed on each node on the network.

Figure 7.10 shows a classification of the different types of attacks possible in ad hoc wireless networks. The following sections describe the various attacks listed in the figure.

Figure 7.10 Classifications of attacks.

7.12.1 Network Layer Attacks

This section lists and gives brief descriptions of the attacks pertaining to the network layer in the network protocol stack.

• Wormhole attack: In this attack, an attacker receives packets at one location in the network and tunnels them (possibly selectively) to another location in the network, where the packets are resent into the network. This tunnel between two colluding attackers is referred to as a wormhole. It could be established through a single long-range wireless link or even through a wired link between the two colluding attackers. Due to the broadcast nature of the radio channel, the attacker can create a wormhole even for packets not addressed to itself. Though no harm is done if the wormhole is used properly for efficient relaying of packets, it puts the attacker in a powerful position compared to other nodes in the network, which the attacker could use in a manner that could compromise the security of the network. If proper mechanisms are not employed to defend the network against wormhole attacks, most of the existing routing protocols for ad hoc wireless networks may fail to find valid routes.

• Blackhole attack: In this attack, a malicious node falsely advertises good paths (e.g., shortest path or most stable path) to the destination node during the path-finding process (in on-demand routing protocols) or in the route update messages (in table-driven routing protocols). The intention of the malicious node could be to hinder the path-finding process or to intercept all data packets being sent to the destination node concerned.

• Byzantine attack: Here, a compromised intermediate node or a set of compromised intermediate nodes works in collusion and carries out attacks such as creating routing loops, routing packets on non-optimal paths, and selectively dropping packets. Byzantine failures are hard to detect. The network would seem to be operating normally in the viewpoint of the nodes, though it may actually be exhibiting Byzantine behavior.

• Information disclosure: A compromised node may leak confidential or important information to unauthorized nodes in the network. Such information may include information regarding the network topology, geographic location of nodes, or optimal routes to authorized nodes in the network.

• Resource consumption attack: In this attack, a malicious node tries to consume/waste away resources of other nodes present in the network. The resources that are targeted are battery power, bandwidth, and computational power, which are only limitedly available in ad hoc wireless networks. The attacks could be in the form of unnecessary requests for routes, very frequent generation of beacon packets, or forwarding of stale packets to nodes. Using up the battery power of another node by keeping that node always busy by continuously pumping packets to that node is known as a sleep deprivation attack.

• Routing attacks: There are several types attacks mounted on the routing protocol which are aimed at disrupting the operation of the network. In what follows, the various attacks on the routing protocol are described briefly.

– Routing table overflow: In this type of attack, an adversary node advertises routes to non-existent nodes, to the authorized nodes present in the network. The main objective of such an attack is to cause an overflow of the routing tables, which would in turn prevent the creation of entries corresponding to new routes to authorized nodes. Proactive routing protocols are more vulnerable to this attack compared to reactive routing protocols.

– Routing table poisoning: Here, the compromised nodes in the networks send fictitious routing updates or modify genuine route update packets sent to other uncompromised nodes. Routing table poisoning may result in sub-optimal routing, congestion in portions of the network, or even make some parts of the network inaccessible.

– Packet replication: In this attack, an adversary node replicates stale packets. This consumes additional bandwidth and battery power resources available to the nodes and also causes unnecessary confusion in the routing process.

– Route cache poisoning: In the case of on-demand routing protocols (such as the AODV protocol, each node maintains a route cache which holds information regarding routes that have become known to the node in the recent past. Similar to routing table poisoning, an adversary can also poison the route cache to achieve similar objectives.

– Rushing attack: On-demand routing protocols that use duplicate suppression during the route discovery process are vulnerable to this attack. An adversary node which receives a RouteRequest packet from the source node floods the packet quickly throughout the network before other nodes which also receive the same RouteRequest packet can react. Nodes that receive the legitimate RouteRequest packets assume those packets to be duplicates of the packet already received through the adversary node and hence discard those packets. Any route discovered by the source node would contain the adversary node as one of the intermediate nodes. Hence, the source node would not be able to find secure routes, that is, routes that do not include the adversary node. It is extremely difficult to detect such attacks in ad hoc wireless networks.

7.12.2 Transport Layer Attacks

This section discusses an attack which is specific to the transport layer in the network protocol stack.

• Session hijacking: Here, an adversary takes control over a session between two nodes. Since most authentication processes are carried out only at the start of a session, once the session between two nodes gets established, the adversary node masquerades as one of the end nodes of the session and hijacks the session.

7.12.3 Application Layer Attacks

This section briefly describes a security flaw associated with the application layer in the network protocol stack.

• Repudiation: In simple terms, repudiation refers to the denial or attempted denial by a node involved in a communication of having participated in all or part of the communication. As mentioned non-repudiation is one of the important requirements for a security protocol in any communication network.

7.12.4 Other Attacks

This section discusses security attacks that cannot strictly be associated with any specific layer in the network protocol stack.

Multi-layer Attacks

Multi-layer attacks are those that could occur in any layer of the network protocol stack. Denial of service and impersonation are some of the common multi-layer attacks. This section discusses some of the multi-layer attacks in ad hoc wireless networks.

• Denial of Service: In this type of attack, an adversary attempts to prevent legitimate and authorized users of services offered by the network from accessing those services. A denial of service (DoS) attack can be carried out in many ways. The classic way is to flood packets to any centralized resource (e.g., an access point) used in the network so that the resource is no longer available to nodes in the network, resulting in the network no longer operating in the manner it was designed to operate.

This may lead to a failure in the delivery of guaranteed services to the end users. Due to the unique characteristics of ad hoc wireless networks, there exist many more ways to launch a DoS attack in such a network, which would not be possible in wired networks. DoS attacks can be launched against any layer in the network protocol stack. On the physical and MAC layers, an adversary could employ jamming signals which disrupt the on-going transmissions on the wireless channel. On the network layer, an adversary could take part in the routing process and exploit the routing protocol to disrupt the normal functioning of the network.

For example, an adversary node could participate in a session but simply drop a certain number of packets, which may lead to degradation in the QoS being offered by the network. On the higher layers, an adversary could bring down critical services such as the key management service (key management will be described in detail in the next section). Some of the DoS attacks are described below.

– Jamming: In this form of attack, the adversary initially keeps monitoring the wireless medium in order to determine the frequency at which the receiver node is receiving signals from the sender. It then transmits signals on that frequency so that error-free reception at the receiver is hindered. Frequency hopping spread spectrum (FHSS) and direct sequence spread spectrum (DSSS) (described in detail in the first chapter of this book) are two commonly used techniques that overcome jamming attacks.

– SYN flooding: Here, an adversary sends a large number of SYN packets8 to a victim node, spoofing the return addresses of the SYN packets. On receiving the SYN packets, the victim node sends back acknowledgment (SYN-ACK) packets to nodes whose addresses have been specified in the received SYN packets.

However, the victim node would not receive any ACK packet in return. In effect, a half-open connection gets created. The victim node builds up a table/data structure for holding information regarding all pending connections. Since the maximum possible size of the table is limited, the increasing number of half-open connections results in an overflow in the table. Hence, even if a connection request comes from a legitimate node at a later point of time, because of the table overflow, the victim node would be forced to reject the call request.

8 SYN packets are used to establish an end-to-end session between two nodes at the transport layer.

– Distributed DoS attack: A more severe form of the DoS attack is the distributed DoS (DDoS) attack. In this attack, several adversaries that are distributed throughout the network collude and prevent legitimate users from accessing the services offered by the network.

• Impersonation: In impersonation attacks, an adversary assumes the identity and privileges of an authorized node, either to make use of network resources that may not be available to it under normal circumstances, or to disrupt the normal functioning of the network by injecting false routing information into the network. An adversary node could masquerade as an authorized node using several methods. It could by chance guess the identity and authentication details of the authorized node (target node), or it could snoop for information regarding the identity and authentication of the target node from a previous communication, or it could circumvent or disable the authentication mechanism at the target node.

A man-in-the-middle attack is another type of impersonation attack. Here, the adversary reads and possibly modifies, messages between two end nodes without letting either of them know that they have been attacked. Suppose two nodes X and Y are communicating with each other; the adversary impersonates node Y with respect to node X and impersonates node X with respect to node Y, exploiting the lack of third-party authentication of the communication between nodes X and Y.

Device Tampering

Unlike nodes in a wired network, nodes in ad hoc wireless networks are usually compact, soft, and hand-held in nature. They could get damaged or stolen easily.

7.13 KEY MANAGEMENT

Having seen the various kinds of attacks possible on ad hoc wireless networks, we now look at various techniques employed to overcome the attacks. Cryptography is one of the most common and reliable means to ensure security. Cryptography is not specific to ad hoc wireless networks. It can be applied to any communication network. It is the study of the principles, techniques, and algorithms by which information is transformed into a disguised version which no unauthorized person can read, but which can be recovered in its original form by an intended recipient. In the parlance of cryptography, the original information to be sent from one person to another is called plaintext. This plaintext is converted into ciphertext by the process of encryption, that is, the application of certain algorithms or functions. An authentic receiver can decrypt/decode the ciphertext back into plaintext by the process of decryption. The processes of encryption and decryption are governed by keys, which are small amounts of information used by the cryptographic algorithms. When the key is to be kept secret to ensure the security of the system, it is called a secret key. The secure administration of cryptographic keys is called key management.

The four main goals of cryptography are confidentiality, integrity, authentication (the receiver should be able to identify the sender and verify that the message actually came from that sender), and non-repudiation. A detailed study of cryptography is presented in.

There are two major kinds of cryptographic algorithms: symmetric key algorithms, which use the same key for encryption and decryption, and asymmetric key algorithms, which use two different keys for encryption and decryption. Symmetric key algorithms are usually faster to execute electronically, but require a secret key to be shared between the sender and receiver. When communication needs to be established among a group of nodes, each sender-receiver pair should share a key, which makes the system non-scalable. If the same key is used among more than two parties, a breach of security at any one point makes the whole system vulnerable.

The asymmetric key algorithms are based on some mathematical principles which make it infeasible or impossible to obtain one key from another; therefore, one of the keys can be made public while the other is kept secret (private). This is called public key cryptography. Such systems are used extensively in practice, but are not provably secure. They rely upon the difficulty of solving certain mathematical problems, and the network would be open to attacks once the underlying mathematical problem is solved.

7.13.1 Symmetric Key Algorithms

Symmetric key algorithms rely on the presence of the shared key at both the sender and receiver, which has been exchanged by some previous arrangement. There are two kinds of symmetric key algorithms, one involving block ciphers and the other stream ciphers. A block cipher is an encryption scheme in which the plaintext is broken into fixed-length segments called blocks, and the blocks are encrypted one at a time. The simplest examples include substitution and transposition. In substitution, each alphabet of the plaintext is substituted by another in the cipher text, and this table mapping the original and the substituted alphabet is available at both the sender and receiver. A transposition cipher permutes the alphabet in the plaintext to produce the cipher text. Figure 7.11(a) illustrates the encryption using substitution, and Figure 7.11 (b) shows a transposition cipher. The block length used is five.

A stream cipher is, in effect, a block cipher of block length one. One of the simplest stream ciphers is the Vernam cipher, which uses a key of the same length as the plaintext for encryption. For example, if the plaintext is the binary string 10010100, and the key is 01011001, then the encrypted string is given by the XOR of the plaintext and key, to be 11001101. The plaintext is again recovered by XORing the cipher text with the same key. If the key is randomly chosen, transported securely to the receiver, and used for only one communication, this forms the one-time pad which has proven to be the most secure of all cryptographic systems. The only bottleneck here is to be able to securely send the key to the receiver.

Figure 7.11. Substitution and transposition.

7.13.2 Asymmetric Key Algorithms

Asymmetric key (or public key) algorithms use different keys at the sender and receiver ends for encryption and decryption, respectively. Let the encryption process be represented by a function E, and decryption by D. Then the plaintext m is transformed into the ciphertext c as c = E(m). The receiver then decodes c by applying D. Hence, D is such that m = D(c) = D(E(m)). When this asymmetric key concept is used in public key algorithms, the key E is made public, while D is private, known only to the intended receiver. Anyone who wishes to send a message to this receiver encrypts it using E. Though ccan be overheard by adversaries, the function E is based on a computationally difficult mathematical problem, such as the factorization of large prime numbers. Hence, it is not possible for adversaries to derive D given E. Only the receiver can decrypt c using the private key D.

A very popular example of public key cryptography is the RSA system developed by Rivest, Shamir, and Adleman, which is based on the integer factorization problem. Digital signatures schemes are also based on public key encryption. In these schemes, the functions Eand D are chosen such that D(E(m)) = E(D(m)) = m for any message m. These are called reversible public key systems. In this case, the person who wishes to sign a document encrypts it using his/her private key D, which is known only to him/her.

Anybody who has his/her public key E can decrypt it and obtain the original document, if it has been signed by the corresponding sender. In practice, a trusted third party (TTP) is agreed upon in advance, who is responsible for issuing these digital signatures (D and E pairs) and for resolving any disputes regarding the signatures. This is usually a governmental or business organization.

7.13.3 Key Management Approaches

The primary goal of key management is to share a secret (some information) among a specified set of participants. There are several methods that can be employed to perform this operation, all of them requiring varying amounts of initial configuration, communication, and computation. The main approaches to key management are key predistribution, key transport, key arbitration, and key agreement.

Key Predistribution

Key predistribution, as the name suggests, involves distributing keys to all interested parties before the start of communication. This method involves much less communication and computation, but all participants must be known a priori, during the initial configuration. Once deployed, there is no mechanism to include new members in the group or to change the key. As an improvement over the basic predistribution scheme, sub-groups may be formed within the group, and some communication can be restricted to a subgroup. However, the formation of sub-groups is also an a priori decision with no flexibility during the operation.

Key Transport

In key transport systems, one of the communicating entities generates keys and transports them to the other members. The simplest scheme assumes that a shared key already exists among the participating members. This prior shared key is used to encrypt a new key and is transmitted to all corresponding nodes. Only those nodes which have the prior shared key can decrypt it. This is called the key encrypting key (KEK) method. However, the existence of a prior key cannot always be assumed. If the public key infrastructure (PKI) is present, the key can be encrypted with each participant's public key and transported to it. This assumes the existence of a TTP, which may not be available for ad hoc wireless networks.

Figure 7.12. Shamir's three-pass protocol.

An interesting method for key transport without prior shared keys is the Shamir's three-pass protocol. The scheme is based on a special type of encryption called commutative encryption schemes [which are reversible and composable (composition of two functions f and g is defined as f(g(x)))]. Consider two nodes X and Y which wish to communicate. Node X selects a key K which it wants to use in its communication with node Y. It then generates another random key kx , using which it encrypts K with  f, and sends to node Y. Node Y encrypts this with a random key ky  using g, and sends it back to node X. Now, node X decrypts this message with its key kx , and after applying the inverse function f-1, sends it to node Y. Finally, node Y decrypts the message using ky and g-1 to obtain the key K. The message exchanges of the protocol are illustrated in Figure 7.12.

Key Arbitration

Key arbitration schemes use a central arbitrator to create and distribute keys among all participants. Hence, they are a class of key transport schemes. Networks which have a fixed infrastructure use the AP as an arbitrator, since it does not have stringent power or computation constraints. In ad hoc wireless networks, the problem with implementation of arbitrated protocols is that the arbitrator has to be powered on at all times to be accessible to all nodes. This leads to a power drain on that particular node. An alternative would be to make the keying service distributed, but simple replication of the arbitration at different nodes would be expensive for resource-constrained devices and would offer many points of vulnerability to attacks. If any one of the replicated arbitrators is attacked, the security of the whole system breaks down.

Key Agreement

Most key agreement schemes are based on asymmetric key algorithms. They are used when two or more people want to agree upon a secret key, which will then be used for further communication. Key agreement protocols are used to establish a secure context over which a session can be run, starting with many parties who wish to communicate and an insecure channel. In group key agreement schemes, each participant contributes a part to the secret key. These need the least amount of pre-configuration, but such schemes have high computational complexity. The most popular key agreement schemes use the Diffie-Hellman exchange, an asymmetric key algorithm based on discrete logarithms.

7.13.4 Key Management in Ad Hoc Wireless Networks

Ad hoc wireless networks pose certain specific challenges in key management due to the lack of infrastructure in such networks. Three types of infrastructure have been identified in, which are absent in ad hoc wireless networks. The first is the network infrastructure, such as dedicated routers and stable links, which ensure communication with all nodes. The second missing infrastructure is services such as name resolution, directory, and TTPs. The third missing infrastructure in ad hoc wireless networks is the administrative support of certifying authorities.

Password-Based Group Systems

Several solutions for group keying in ad hoc wireless networks have been suggested in. The example scenario for implementation is a meeting room, where different mobile devices want to start a secure session. Here, the parties involved in the session are to be identified based on their location, that is, all devices in the room can be part of the session. Hence, relative location is used as the criterion for access control. If a TTP which knows the location of the participants exists, then it can implement location-based access control. A prior shared secret can be obtained by a physically more secure medium such as a wired network. This secret can be obtained by plugging onto a wired network first, before switching to the wireless mode.

A password-based system has been explored where, in the simplest case, a long string is given as the password for users for one session. However, human beings tend to favor natural language phrases as passwords, over randomly generated strings. Such passwords, if used as keys directly during a session, are very weak and open to attack because of high redundancy, and the possibility of reuse over different sessions. Hence, protocols have been proposed to derive a strong key (not vulnerable to attacks) from the weak passwords given by the participants.

This password-based system could be two-party, with a separate exchange between any two participants, or it could be for the whole group, with a leader being elected to preside over the session. Leader election is a special case of establishing an order among all participants. The protocol used is as follows. Each participant generates a random number, and sends it to all others. When every node has received the random number of every other node, a common pre-decided function is applied on all the numbers to calculate a reference value. The nodes are ordered based on the difference between their random number and the reference value.

Threshold Cryptography

Public key infrastructure (PKI) enables the easy distribution of keys and is a scalable method. Each node has a public/private key pair, and a certifying authority (CA) can bind the keys to the particular node. But the CA has to be present at all times, which may not be feasible in ad hoc wireless networks. It is also not advisable to simply replicate the CA at different nodes. In [20], a scheme based on threshold cryptography has been proposed by which n servers exist in the ad hoc wireless network, out of which any (t+1) servers can jointly perform any arbitration or authorization successfully, but t servers cannot perform the same.

Hence, up to t compromised servers can be tolerated. This is called an (n, t + 1) configuration, where n≥3t+1. To sign a certificate, each server generates a partial signature using its private key and submits it to a combiner. The combiner can be any one of the servers. In order to ensure that the key is combined correctly, t + 1 combiners can be used to account for at most t malicious servers. Using t + 1 partial signatures (obtained from itself and t other servers), the combiner computes a signature and verifies its validity using a public key. If the verification fails, it means that at least one of the t + 1 keys is not valid, so another subset of t + 1 partial signatures is tried. If the combiner itself is malicious, it cannot get a valid key, because the partial signature of itself is always invalid.

The scheme can be applied to asynchronous networks, with no bound on message delivery or processing times. This is one of the strengths of the scheme, as the requirement of synchronization makes the system vulnerable to DoS attacks. An adversary can delay a node long enough to violate the synchrony assumption, thereby disrupting the system.

Sharing a secret in a secure manner alone does not completely fortify a system. Mobile adversaries can move from one server to another, attack them, and get hold of their private keys. Over a period of time, an adversary can have more than t private keys. To counter this, share refreshing has been proposed, by which servers create a new independent set of shares (the partial signatures which are used by the servers) periodically. Hence, to break the system, an adversary has to attack and capture more than tservers within the period between two successive refreshes; otherwise, the earlier share information will no longer be valid. This improves protection against mobile adversaries.

Self-Organized Public Key Management for Mobile Ad Hoc Networks

The authors of [24] have proposed a completely self-organized public key system for ad hoc wireless networks. This makes use of absolutely no infrastructure – TTP, CA, or server – even during initial configuration. The users in the ad hoc wireless network issue certificates to each other based on personal acquaintance. A certificate is a binding between a node and its public key. These certificates are also stored and distributed by the users themselves. Certificates are issued only for a specified period of time and contain their time of expiry along with them. Before it expires, the certificate is updated by the user who had issued the certificate.

Initially, each user has a local repository consisting of the certificates issued by him and the certificates issued by other users to him. Hence, each certificate is initially stored twice, by the issuer and by the person for whom it is issued. Periodically, certificates from neighbors are requested and the repository is updated by adding any new certificates. If any of the certificates are conflicting (e.g., the same public key to different users, or the same user having different public keys), it is possible that a malicious node has issued a false certificate. A node then labels such certificates as conflicting and tries to resolve the conflict. Various methods exist to compare the confidence in one certificate over another. For instance, another set of certificates obtained from another neighbor can be used to take a majority decision. This can be used to evaluate the trust in other users and detect malicious nodes. If the certificates issued by some node are found to be wrong, then that node may be assumed to be malicious.       	The authors of [24] define a certificate graph as a graph whose vertices are public keys of some nodes and whose edges are public-key certificates issued by users. When a user X wants to obtain the public key of another user Y, he/she finds a chain of valid public key certificates leading to Y. The chain is such that the first hop uses an edge from X, that is, a certificate issued by X, the last hop leads into Y(this is a certificate issued to Y), and all intermediate nodes are trusted through the previous certificate in the path. The protocol assumes that trust is transitive, which may not always be valid.

Having seen the various key management techniques employed in ad hoc wireless networks, we now move on to discuss some of the security-aware routing schemes for ad hoc wireless networks.

7.14 SECURE ROUTING IN AD HOC WIRELESS NETWORKS

Unlike the traditional wired Internet, where dedicated routers controlled by the Internet service providers (ISPs) exist, in ad hoc wireless networks, nodes act both as regular terminals (source or destination) and also as routers for other nodes. In the absence of dedicated routers, providing security becomes a challenging task in these networks. Various other factors which make the task of ensuring secure communication in ad hoc wireless networks difficult include the mobility of nodes, a promiscuous mode of operation, limited processing power, and limited availability of resources such as battery power, bandwidth, and memory. In the following sections, we show how some of the well-known traditional routing protocols for ad hoc networks fail to provide security. Some of the mechanisms proposed for secure routing are also discussed.

7.14.1 Requirements of a Secure Routing Protocol for Ad Hoc Wireless Networks

The fundamental requisites of a secure routing protocol for ad hoc wireless networks are listed as follows:

• Detection of malicious nodes: A secure routing protocol should be able to detect the presence of malicious nodes in the network and should avoid the participation of such nodes in the routing process. Even if such malicious nodes participate in the route discovery process, the routing protocol should choose paths that do not include such nodes.

• Guarantee of correct route discovery: If a route between the source and the destination nodes exists, the routing protocol should be able to find the route, and should also ensure the correctness of the selected route.

• Confidentiality of network topology: an information disclosure attack may lead to the discovery of the network topology by the malicious nodes. Once the network topology is known, the attacker may try to study the traffic pattern in the network. If some of the nodes are found to be more active compared to others, the attacker may try to mount (e.g., DoS) attacks on such bottleneck nodes. This may ultimately affect the on-going routing process. Hence, the confidentiality of the network topology is an important requirement to be met by the secure routing protocols.

• Stability against attacks: The routing protocol must be self-stable in the sense that it must be able to revert to its normal operating state within a finite amount of time after a passive or an active attack. The routing protocol should take care that these attacks do not permanently disrupt the routing process. The protocol must also ensure Byzantine robustness, that is, the protocol should work properly even if some of the nodes, which were earlier participating in the routing process, turn out to become malicious at a later point of time or are intentionally damaged.

7.14.2 Security-Aware Ad Hoc Routing Protocol

The security-aware ad hoc routing (SAR) protocol uses security as one of the key metrics in path finding. A framework for enforcing and measuring the attributes of the security metric has been provided in [25]. This framework also enables the use of different levels of security for different applications that use SAR for routing. In ad hoc wireless networks, communication between end nodes through possibly multiple intermediate nodes is based on the fact that the two end nodes trust the intermediate nodes.

SAR defines level of trust as a metric for routing and as one of the attributes for security to be taken into consideration while routing. The routing protocol based on the level of trust is explained using Figure 7.13. As shown in Figure 7.13, two paths exist between the two officers O1 andO2 who want to communicate with each other. One of these paths is a shorter path which runs through private nodes whose trust levels are very low. Hence, the protocol chooses a longer but secure path which passes through other secure (officer) nodes.

Figure 7.13. Illustration of the level of trust metric.

The SAR protocol can be explained using any one of the traditional routing protocols. This section explains SAR using the AODV protocol. In the AODV protocol, the source node broadcasts a RouteRequest packet to its neighbors. An intermediate node, on receiving a RouteRequest packet, forwards it further if it does not have a route to the destination. Otherwise, it initiates a RouteReply packet back to the source node using the reverse path traversed by the RouteRequest packet.

In SAR, a certain level of security is incorporated into the packet-forwarding mechanism. Here, each packet is associated with a security level which is determined by a number calculation method (explained later in this section). Each intermediate node is also associated with a certain level of security. On receiving a packet, the intermediate node compares its level of security with that defined for the packet. If the node's security level is less than that of the packet, the RouteRequest is simply discarded. If it is greater, the node is considered to be a secure node and is permitted to forward the packet in addition to being able to view the packet. If the security levels of the intermediate node and the received packet are found to be equal, then the intermediate node will not be able to view the packet (which can be ensured using a proper authentication mechanism); it just forwards the packet further.

Nodes of equal levels of trust distribute a common key among themselves and with those nodes having higher levels of trust. Hence, a hierarchical level of security could be maintained. This ensures that an encrypted packet can be decrypted (using the common key) only by nodes of the same or higher levels of security compared to the level of security of the packet. Different levels of trust can be defined using a number calculated based on the level of security required. It can be calculated using many methods. Since timeliness, in-order delivery of packets, authenticity, authorization, integrity, confidentiality, and non-repudiation are some of the desired characteristics of a routing protocol, a suitable number can be defined for the trust level for nodes and packets based on the number of such characteristics taken into account.

The SAR mechanism can be easily incorporated into the traditional routing protocols for ad hoc wireless networks. It could be incorporated into both on-demand and table-driven routing protocols. The SAR protocol allows the application to choose the level of security it requires. But the protocol requires different keys for different levels of security. This tends to increase the number of keys required when the number of security levels used increases.

7.14.3 Secure Efficient Ad Hoc Distance Vector Routing Protocol

Secure efficient ad hoc distance vector (SEAD) routing protocol, is a secure ad hoc routing protocol based on the destination-sequenced distance vector (DSDV) routing protocol. This protocol is mainly designed to overcome security attacks such as DoS and resource consumption attacks. The operation of the routing protocol does not get affected even in the presence of multiple uncoordinated attackers corrupting the routing tables. The protocol uses a one-way hash function and does not involve any asymmetric cryptographic operation.

Distance Vector Routing

Distance vector routing protocols belong to the category of table-driven routing protocols. Each node maintains a routing table containing the list of all known routes to various destination nodes in the network. The metric used for routing is the distance measured in terms of hop-count. The routing table is updated periodically by exchanging routing information. An alternative to this approach is triggered updates, in which each node broadcasts routing updates only if its routing table gets altered.

The DSDV protocol for ad hoc wireless networks uses sequence number tags to prevent the formation of loops, to counter the count-to-infinity problem, and for faster convergence. When a new route update packet is received for a destination, the node updates the corresponding entry in its routing table only if the sequence number on the received update is greater than that recorded with the corresponding entry in the routing table.

If the received sequence number and the previously recorded sequence number are both equal, but if the routing update has a new value for the routing metric (distance in number of hops), then in this case also the update is effected. Otherwise, the received update packet is discarded. DSDV uses triggered updates (for important routing changes) in addition to the regular periodic updates. A slight variation of DSDV protocol known as DSDV-SQ (DSDV for sequence numbers) initiates triggered updates on receiving a new sequence number update.

One-Way Hash Function

SEAD uses authentication to differentiate between updates that are received from non-malicious nodes and malicious nodes. This minimizes resource consumption attacks caused by malicious nodes. SEAD uses a one-way hash function for authenticating the updates. A one-way hash function (H) generates a one- way hash chain (h1 , h2 , ...). The function H maps an input bit-string of any length to a fixed length bit-string, that is, H p, where p is the length in bits of the output bit-string. To create a one-way hash chain, a node generates a random number with initial value x€(0, 1)p. h0 , the first number in the hash chain is initialized to x. The remaining values in the chain are computed using the general formula, hi = H(hi- 1) for 0≤i≤n, for some n.

Now we shall see how the one-way hash function incorporates security into the existing DSDV-SQ routing protocol. The SEAD protocol assumes an upper bound on the metric used. For example, if the metric used is distance, then the upper bound value m - 1 defines the maximum diameter (maximum of lengths of all the routes between a pair of nodes) of the ad hoc wireless network. Hence, the routing protocol ensures that no route of length greater than m hops exists between any two nodes.

If the sequence of values calculated by a node using the hash function H is given by (h1 , h2 , ..., hn ), where n is divisible by m, then for a routing table entry with sequence number i, let . If the metric j (distance) used for that routing table entry is 0  - 1, then the value hkm+j  is used to authenticate the routing update entry for that sequence number i and that metric j. Whenever a route update message is sent, the node appends the value used for authentication along with it. If the authentication value used is hkm+j , then the attacker who tries to modify this value can do so only if he/she knows hkm+j- 1.

Since it is a one-way hash chain, calculating hkm+j- 1 becomes impossible. An intermediate node, on receiving this authenticated update, calculates the new hash value based on the earlier updates (hkm+j- 1 ), the value of the metric, and the sequence number. If the calculated value matches with the one present in the route update message, then the update is effected; otherwise, the received update is just discarded. SEAD avoids routing loops unless the loop contains more than one attacker. This protocol could be implemented easily with slight modifications to the existing distance vector routing protocols. The protocol is robust against multiple uncoordinated attacks. The SEAD protocol, however, would not be able to overcome attacks where the attacker uses the same metric and sequence number which were used by the recent update message, and sends a new routing update.

7.14.4 Authenticated Routing for Ad Hoc Networks

Authenticated routing for ad hoc networks (ARAN) routing protocol, based on cryptographic certificates, is a secure routing protocol which successfully defeats all identified attacks in the network layer. It takes care of authentication, message integrity, and non-repudiation, but expects a small amount of prior security coordination among nodes. In [28], vulnerabilities and attacks specific to AODV and DSR protocols are discussed and the two protocols are compared with the ARAN protocol.

During the route discovery process of ARAN, the source node broadcasts RouteRequest packets. The destination node, on receiving the RouteRequest packets, responds by unicasting back a reply packet on the selected path. The ARAN protocol uses a preliminary cryptographic certification process, followed by an end-to-end route authentication process, which ensures secure route establishment.

Issue of Certificates

This section discusses the certification process in which the certificates are issued to the nodes in the ad hoc wireless network. There exists an authenticated trusted server whose public key is known to all legal nodes in the network. The ARAN protocol assumes that keys are generated a priori by the server and distributed to all nodes in the network. The protocol does not specify any specific key distribution algorithm. On joining the network, each node receives a certificate from the trusted server. The certificate received by a node A from the trusted server T looks like the following:

(9.12.1)

Here, IPA , KA+, t, e, and KT - represent the IP address of node A, the public key of node A, the time of creation of the certificate, the time of expiry of the certificate, and the private key of the server, respectively.

End-to-End Route Authentication

The main goal of this end-to-end route authentication process is to ensure that the correct intended destination is reached by the packets sent from the source node. The source node S broadcasts a RouteRequest/RouteDiscovery packet destined to the destination node D. The RouteRequest packet contains the packet identifier [route discovery process (RDP)], the IP address of the destination (IPD ), the certificate of the source node S (CertS), the current time (t), and nonce NS . The process can be denoted as below. Here, KS - is the private key of the source node S.

(9.12.2)

Whenever the source sends a route discovery message, it increments the value of nonce. Nonce is a counter used in conjunction with the time-stamp in order to make the nonce recycling easier. When a node receives an RDP packet from the source with a higher value of the source's nonce than that in the previously received RDP packets from the same source node, it makes a record of the neighbor from which it received the packet, encrypts the packet further with its own certificate, and broadcasts it further. The process can be denoted as follows:

(9.12.3)

An intermediate node B, on receiving an RDP packet from a node A, removes its neighbor's certificate, inserts its own certificate, and broadcasts the packet further. The destination node, on receiving an RDP packet, verifies node S's certificate and the tuple (NS,t) and then replies with the RouteReply packet (REP). The destination unicasts the REP packet to the source node along the reverse path as follows:

(9.12.4)

where node X is the neighbor of the destination node D, which had originally forwarded the RDP packet to node D. The REP packet follows the same procedure on the reverse path as that followed by the route discovery packet. An error message is generated if the time-stamp or nonce do not match the requirements or if the certificate fails. The error message looks similar to the other packets except that the packet identifier is replaced by the ERR message.

Table 7.1 shows a comparison between the AODV, DSR, and ARAN protocols with respect to their security-related features. ARAN remains robust in the presence of attacks such as unauthorized participation, spoofed route signaling, fabricated routing messages, alteration of routing messages, securing shortest paths, and replay attacks.

Table 7.1. Comparison of vulnerabilities of ARAN with DSR and AODV protocols

7.14.5 Security-Aware AODV Protocol

This section discusses security solutions that address a particular security flaw in the AODV routing protocol. AODV is an on-demand routing protocol where the route discovery process is initiated by sending RouteRequest packets only when data packets arrive at a node for transmission. A malicious intermediate node could advertise that it has the shortest path to the destination, thereby redirecting all the packets through itself. This is known as a black hole attack. The blackhole attack is illustrated in Figure 7.14. Let node M be the malicious node that enters the network. It advertises that it has the shortest path to the destination node D when it receives the RouteRequest packet sent by node S.

The attacker may not be able to succeed if node A, which also receives the RouteRequest packet from node S, replies earlier than node M. But a major advantage for the malicious node is that it does not have to search its routing table for a route to the destination. Also, the RouteReply packets originate directly from the malicious node and not from the destination node. Hence, the malicious node would be able to reply faster than node A, which would have to search its routing table for a route to the destination node. Thus, node S may tend to establish a route to destination D through the malicious node M, allowing node M to listen to all packets meant for the destination node.

Figure 7.14. Illustration of blackhole problem.

Solutions for the Blackhole Problem

One of the solutions for the blackhole problem is to restrict the intermediate nodes from originatingRouteReply packets. Only the destination node would be permitted to initiate RouteReply packets. Security is still not completely assured, since the malicious node may lie in the path chosen by the destination node. Also, the delay involved in the route discovery process increases as the size of the network increases. In another solution to this problem, suggested in [29], as soon as the RouteReplypacket is received from one of the intermediate nodes, another RouteRequest packet is sent from the source node to the neighbor node of the intermediate node in the path. This is to ensure that such a path exists from the intermediate node to the destination node. For example, let the source node send RouteRequest packets and receive RouteReply through the intermediate malicious node M. The RouteReply packet of node M contains information regarding its next-hop neighbor nodes. Let it contain information about the neighbor node E. Then, as shown in Figure 7.15, the source node S sends Further RouteRequest packets to this neighbor node E. Node E responds by sending a FurtherRouteReply packet to source node S.

Since node M is a malicious node which is not present in the routing list of node E, the FurtherRouteReply packet sent by node E will not contain a route to the malicious node M. But if it contains a route to the destination node D, then the new route to the destination through node E is selected, and the earlier selected route through node M is rejected. This protocol completely eliminates the blackhole attack caused by a single attacker. The major disadvantage of this scheme is that the control overhead of the routing protocol increases considerably. Also, if the malicious nodes work in a group, this protocol fails miserably.

Figure 7.15. Propagation of FurtherRouteRequest  and  FurtherRouteReply.

8.1 Sensor Node Hardware

Sensor node hardware can be grouped into three categories, each of which entails a different set of trade-offs in the design choices.

• Augmented general-purpose computers: Examples include low power PCs, embedded PCs (e.g., PC104), custom-designed PCs (e.g., Sensoria WINS NG nodes),1 and various personal digital assistants (PDA). These nodes typically run off-the-shelf operating systems such as Win CE, Linux, or real-time operating systems and use standard wireless communication protocols such as Bluetooth or IEEE 802.11. Because of their relatively higher processing capability, they can accommodate a wide variety of sensors, ranging from simple microphones to more sophisticated video cameras.

Compared with dedicated sensor nodes, PC-like platforms are more power hungry. However, when power is not an issue, these platforms have the advantage that they can leverage the availability of fully supported networking protocols, popular programming languages, middleware, and other off-the-shelf software.

• Dedicated embedded sensor nodes: Examples include the Berkeley mote family, the UCLA Medusa family, Ember nodes,2 and MIT μAMP. These platforms typically use commercial off-the-shelf (COTS) chip sets with emphasis on small form factor, low power processing and communication, and simple sensor interfaces. Because of their COTS CPU, these platforms typically support at least one programming language, such as C. However, in order to keep the program footprint small to accommodate their small memory size, programmers of these platforms are given full access to hardware but barely any operating system support.

A classical example is the Tiny OS platform and its companion programming language, nesC. We will discuss these platforms in Sections 8.3.1 and 8.3.2.

• System-on-chip (SoC) nodes: Examples of SoC hardware include smart dust, the BWRC picoradio node, and the PASTA node.3 Designers of these platforms try to push the hardware limits by fundamentally rethinking the hardware architecture trade-offs for a sensor node at the chip design level. The goal is to find new ways of integrating CMOS, MEMS, and RF technologies to build extremely low power and small footprint sensor nodes that still provide certain sensing, computation, and communication capabilities. Since most of these platforms are currently in the research pipeline with no predefined instruction set, there is no software platform support available.

Among these hardware platforms, the Berkeley motes, due to their small form factor, open source software development, and commercial availability, have gained wide popularity in the sensor network research community. In the following section, we give an overview of the Berkeley MICA mote.

8.1.1 Berkeley Motes

The Berkeley motes are a family of embedded sensor nodes sharing roughly the same architecture. Figure 8.1 shows a comparison of a subset of mote types. Let us take the MICA mote as an example. The MICA motes have a two-CPU design, as shown in Figure 8.2. The main microcontroller (MCU), an Atmel ATmega103L, takes care of regular processing. A separate and much less capable coprocessor is only active when the MCU is being reprogrammed.

The ATmega103L MCU has integrated 512 KB flash memory and 4 KB of data memory. Given these small memory sizes, writing software for motes is challenging. Ideally, programmers should be relieved from optimizing code at assembly level to keep code footprint small. However, high-level support and software services are not free. Being able to mix and match only necessary software components to support a particular application is essential to achieving a small footprint. A detailed discussion of the software architecture for motes is given in Section 8.3.1.

Figure 8.1 A comparison of Berkeley motes.

Figure 8.2 MICA mote architecture.

In addition to the memory inside the MCU, a MICA mote also has a separate 512 KB flash memory unit that can hold data. Since the connection between the MCU and this external memory is via a low-speed serial peripheral interface (SPI) protocol, the external memory is more suited for storing data for later batch processing than for storing programs. The RF communication on MICA motes uses the TR1000 chip set (from RF Monolithics, Inc.) operating at 916 MHz band. With hardware accelerators, it can achieve a maximum of 50 kbps raw data rate. MICA motes implement a 40 kbps transmission rate. The transmission power can be digitally adjusted by software though a potentiometer (Maxim DS1804). 	The maximum transmission range is about 300 feet in open space. Like other types of motes in the family, MICA motes support a 51 pin I/O extension connector. Sensors, actuators, serial I/O boards, or parallel I/O boards can be connected via the connector. A sensor/actuator board can host a temperature sensor, a light sensor, an accelerometer, a magnetometer, a microphone, and a beeper. The serial I/O (UART) connection allows the mote to communicate with a PC in real time. The parallel connection is primarily for downloading programs to the mote. It is interesting to look at the energy consumption of various components on a MICA mote. As shown in Figure 8.3, a radio transmission bears the maximum power consumption. However, each radio packet (e.g., 30 bytes) only takes 4 ms to send, while listening to incoming packets turns the radio receiver on all the time.

Figure 8.3 Power consumption of MICA motes.

The energy that can send one packet only supports the radio receiver for about 27 ms. Another observation is that there are huge differences among the power consumption levels in the active mode, the idle mode, and the suspend mode of the MCU. It is thus worthwhile from an energy-saving point of view to suspend the MCU and the RF receiver as long as possible.

8.2 Sensor Network Programming Challenges

Traditional programming technologies rely on operating systems to provide abstraction for processing, I/O, networking, and user interaction hardware, as illustrated in Figure 8.4. When applying such a model to programming networked embedded systems, such as sensor networks, the application programmers need to explicitly deal with message passing, event synchronization, interrupt handing, and sensor reading.

As a result, an application is typically implemented as a finite state machine (FSM) that covers all extreme cases: unreliable communication channels, long delays, irregular arrival of messages, simultaneous events, and so on. In a target tracking application implemented on a Linux operating system and with directed diffusion routing, roughly 40 percent of the code implements the FSM and the glue logic of interfacing computation and communication.

For resource-constrained embedded systems with real-time requirements, several mechanisms are used in embedded operating systems to reduce code size, improve response time, and reduce energy consumption. Microkernel technologies modularize the operating system so that only the necessary parts are deployed with the application. Real-time scheduling allocates resources to more urgent tasks so that they can be finished early.

Figure 8.4 Traditional embedded system programming interface

Event-driven execution allows the system to fall into low-power sleep mode when no interesting events need to be processed. At the extreme, embedded operating systems tend to expose more hardware controls to the programmers, who now have to directly face device drivers and scheduling algorithms, and optimize code at the assembly level. Although these techniques may work well for small, stand-alone embedded systems, they do not scale up for the programming of sensor networks for two reasons.

• Sensor networks are large-scale distributed systems, where global properties are derivable from program execution in a massive number of distributed nodes. Distributed algorithms themselves are hard to implement, especially when infrastructure support is limited due to the ad hoc formation of the system and constrained power, memory, and bandwidth resources.

• As sensor nodes deeply embed into the physical world, a sensor network should be able to respond to multiple concurrent stimuli at the speed of changes of the physical phenomena of interest. In the rest of the chapter, we give several examples of sensor network software design platforms. We discuss them in terms of both design methodologies and design platforms. A design methodology implies a conceptual model for programmers, with associated techniques for problem decomposition for the software designers.

For example, does the programmer think in terms of events, message passing, and synchronization, or does he/she focus more on information architecture and data semantics? A design platform supports a design methodology by providing design-time (precompile time) language constructs and restrictions, and run-time (postcompile time) execution services. There is no single universal design methodology for all applications. Depending on the specific tasks of a sensor network and the way the sensor nodes are organized, certain methodologies and platforms may be better choices than others. For example, if the network is used for monitoring a small set of phenomena and the sensor nodes are organized in a simple star topology, then a client-server software model would be sufficient.

If the network is used for monitoring a large area from a single access point (i.e., the base station), and if user queries can be decoupled into aggregations of sensor readings from a subset of sensor nodes, then a tree structure that is rooted at the base station is a better choice. However, if the phenomena to be monitored are moving targets, then neither the simple client-server model nor the tree organization is optimal. More sophisticated design methodologies and platforms are required.

8.3 Node-Level Software Platforms

Most design methodologies for sensor network software are node centric, where programmers think in terms of how a node should behave in the environment. A node-level platform can be a node centric operating system, which provides hardware and networking abstractions of a sensor node to programmers, or it can be a language platform, which provides a library of components to programmers. A typical operating system abstracts the hardware platform by providing a set of services for applications, including file management, memory allocation, task scheduling, peripheral device drivers, and networking. For embedded systems, due to their highly specialized applications and limited resources, their operating systems make different trade-offs when providing these services. For example, if there is no file management requirement, then a file system is obviously not needed. If there is no dynamic memory allocation, then memory management can be simplified. If prioritization among tasks is

critical, then a more elaborate priority scheduling mechanism may be added.

TinyOS and TinyGALS are two representative examples of node-level programming tools that we will cover in detail in this section. Other related software platforms include Maté, a virtual machine for the Berkeley motes. Observing that operations such as polling sensors and accessing internal states are common to all sensor network application, Maté defines virtual machine instructions to abstract those operations. When a new hardware platform is introduced with support for the virtual machine, software written in the Maté instruction set does not have to be rewritten.

8.3.1 Operating System: TinyOS

TinyOS aims at supporting sensor network applications on resource constrained hardware platforms, such as the Berkeley motes. To ensure that an application code has an extremely small footprint, TinyOS chooses to have no file system, supports only static memory allocation, implements a simple task model, and provides minimal device and networking abstractions. Furthermore, it takes a language-based application development approach, to be discussed later, so that only the necessary parts of the operating system are compiled with the application. To a certain extent, each TinyOS application is built into the operating system.

Like many operating systems, TinyOS organizes components into layers. Intuitively, the lower a layer is, the “closer” it is to the hardware; the higher a layer is, the “closer” it is to the application. In addition to the layers, TinyOS has a unique component architecture and provides as a library a set of system software components. A component specification is independent of the component implementation. Although most components encapsulate software functionalities, some are just thin wrappers around hardware. An application, typically developed in the nesC language covered in the next section, wires these components together with other application-specific components. Let us consider a TinyOS application example—FieldMonitor, where all nodes in a sensor field periodically send their temperature and photo sensor readings to a base station via an ad hoc routing mechanism. A diagram of the Field Monitor application is shown in Figure 8.5, where blocks represent TinyOS components and arrows represent function calls among them. The directions of the arrows are from callers to callees.

To explain in detail the semantics of TinyOS components, let us first look at the Timer component of the Field Monitor application, as shown in Figure 8.6. This component is designed to work with a clock, which is a software wrapper around a hardware clock that generates periodic interrupts. The method calls of the Timer component are shown in the figure as the arrowheads. An arrowhead pointing into the component is a method of the component that other components can call. An arrowhead pointing outward is a method that this component requires another layer component to provide. The absolute directions of the arrows, up or down, illustrate this component’s relationship with other layers. For example, the Timer depends on a lower layer HWClock component.

Figure 8.5 The FieldMonitor application for sensing and sending measurements.

Figure 8.6 The Timer component and its interfaces.

The Timer can set the rate of the clock, and in response to each clock interrupt it toggles an internal Boolean flag, even Flag, between true (or 1) and false (or 0). If the flag is 0, the Timer produces a timer0Fire event to trigger other components; otherwise, it produces a timer1Fire event. The Timer has an init() method that initializes its internal flag, and it can be enabled and disabled via the start and stop calls. A program executed in TinyOS has two contexts, tasks and events, which provide two sources of concurrency. Tasks are created (also called posted) by components to a task scheduler. The default implementation of the TinyOS scheduler maintains a task queue and invokes tasks according to the order in which they were posted.

Thus tasks are deferred computation mechanisms. Tasks always run to completion without preempting or being preempted by other tasks. Thus tasks are non-preemptive. The scheduler invokes a new task from the task queue only when the current task has completed. When no tasks are available in the task queue, the scheduler puts the CPU into the sleep mode to save energy.

The ultimate sources of triggered execution are events from hardware: clock, digital inputs, or other kinds of interrupts. The execution of an interrupt handler is called an event context. The processing of events also runs to completion, but it preempts tasks and can be preempted by other events. Because there is no preemption mechanism among tasks and because events always preempt tasks, programmers are required to chop their code, especially the code in the event contexts, into small execution pieces, so that it will not block other tasks for too long.

Another trade-off between non-preemptive task execution and program reactiveness is the design of split-phase operations in TinyOS. Similar to the notion of asynchronous method calls in distributed computing, a split-phase operation separates the initiation of a method call from the return of the call. A call to a split phase operation returns immediately, without actually performing the body of the operation. The true execution of the operation is scheduled later; when the execution of the body finishes, the operation notifies the original caller through a separate method call.

An example of a split-phase operation is the packet send method in the Active Messages (AM) component, used in Figure 8.5. Sending a packet is a long operation, involving converting the packets to bytes, then to bits, and ultimately driving the RF circuits to send the bits one by one. Without a split-phase execution, sending a packet will block the entire system from reacting to new events for a significant period of time. In the TinyOS implementation, the send() command in the AM component returns immediately.

However, it is the caller’s responsibility to remember that the packet has not yet been sent. When the packet is indeed sent, the AM component will notify its caller by a sendDone() method call. Only at this time is the AM component ready to accept another packet. In TinyOS, resource contention is typically handled through explicit rejection of concurrent requests. All split-phase operations return Boolean values indicating whether a request to perform the operation is accepted. In the above example, a call of send(), when the AM component is still sending the first packet, will result in an error signaled by the AM component. To avoid such an error, the caller of the AM component typically implements a pending lock, to remember not to request further sendings until the sendDone() method is called. To avoid loss of packets, a queue should be incorporated by the caller if necessary.

In summary, many design decisions in TinyOS are made to ensure that it is extremely lightweight. Using a component architecture that contains all variables inside the components and disallowing dynamic memory allocation reduces the memory management overhead and makes the data memory usage statically analyzable. The simple concurrency model allows high concurrency with low thread maintenance overhead. As a consequence, the entire FieldMonitor system shown in Figure 8.5 takes only 3 KB of space for code and 226 bytes for data. However, the advantage of being lightweight is not without cost.

Many hardware idiosyncrasies and complexities of concurrency management are left for the application programmers to handle. Several tools have been developed to give programmers language-level support for improving programming productivity and code robustness. We introduce in the next two sections two special purpose languages for programming sensor network nodes. Although both languages are designed on top of TinyOS, the principles they represent may apply to other platforms.

8.3.2 Imperative Language: nesC

nesC is an extension of C to support and reflect the design of TinyOS v1.0 and above. It provides a set of language constructs and restrictions to implement TinyOS components and applications. Component Interface

A component in nesC has an interface specification and an implementation. To reflect the layered structure of TinyOS, interfaces of a nesC component are classified as provides or uses interfaces. A provides interface is a set of method calls exposed to the upper layers, while a uses interface is a set of method calls hiding the lower layer components. Methods in the interfaces can be grouped and named. For example, the interface specification of the Timer component in Figure 8.6 is listed in Figure 8.7. The interface, again, independent of the implementation, is called TimerModule. Although they have the same method call semantics, nesC distinguishes the directions of the interface calls between layers as event calls and command calls.

Figure 8.7 The interface definition of the Timer component in nesC.

An event call is a method call from a lower layer component to a higher layer component, while a command is the opposite. Note that one needs to know both the type of the interface (provides or uses) and the direction of the method call (event or command) to know exactly whether an interface method is implemented by the component or is required by the component.

The separation of interface type definitions from how they are used in the components promotes the reusability of standard interfaces. A component can provide and use the same interface type, so that it can act as a filter interposed between a client and a service. A component may even use or provide the same interface multiple times. In these cases, the component must give each interface instance a separate name using the as notation, as shown in the Clock interface in Figure 8.7.

Component Implementation

There are two types of components in nesC, depending on how they are implemented: modules and configurations. Modules are implemented by application code (written in a C-like syntax). Configurations are implemented by connecting interfaces of existing components. The implementation part of a module is written in C-like code. A command or an event bar in an interface foo is referred as foo.bar. A keyword call indicates the invocation of a command. A keyword signal indicates the triggering by an event. For example, Figure 7.8 shows part of the implementation of the Timer component, whose interface is defined in Figure 7.7. In a sense, this implementation is very much like an object in object-oriented programming without any constructors.

Configuration is another kind of implementation of components, obtained by connecting existing components. Suppose we want to connect the Timer component and a hardware clock wrapper, called HWClock, to provide a timer service, called TimerC. Figure 7.9 shows a conceptual diagram of how the components are connected, and Figure 7.10 shows the corresponding nesC code.

Figure 8.8 The implementation definition of the Timer component in nesC.

Figure 8.9 The TimerC configuration implemented by connecting Timer with HWClock.

First of all, notice that the keyword configuration in the specification indicates that this component is not implemented directly as a module. In the implementation section of the configuration, the code first includes the two components, and then specifies that the interface StdControl of the TimerC component is the StdControl interface of the TimerModule; similarly for the Timer01 interface. The connection between the Clock interfaces is specified using the -> operator. Essentially, this interface is hidden from upper layers. nesC also supports the creation of several instances of a component by declaring abstract components with optional parameters. Abstract components are created at compile time in configurations.

Recall that TinyOS does not support dynamic memory allocation, so all components are statically constructed at compile time. A complete application is always a configuration rather than a module. An application must contain the Main module, which links the code to the scheduler at run time. The Main has a single StdControl interface, which is the ultimate source of initialization of all components.

Concurrency and Atomicity

The language nesC directly reflects the TinyOS execution model through the notion of command and event contexts. Figure 8.11 shows a section of the component SenseAndSend to illustrate some language features to support concurrency in nesC and the effort to reduce race conditions.

Figure 8.10 The implementation definition of the TimerC configuration in nesC.

The SenseAndSend component is intended to be built on top of the Timer component, an ADC component, which can provide sensor readings, and a communication component, which can send (or, more precisely, broadcast) a packet. When responding to a timer0Fire event, the SenseAndSend component invokes the ADC to poll a sensor reading. Since polling a sensor reading can take a long time, a split-phase operation is implemented for getting sensor readings.

The call to ADC.getData() returns immediately, and the completion of the operation is signaled by an ADC.dataReady() event. A busy flag is used to explicitly reject new requests while the ADC is fulfilling an existing request. The ADC.getData() method sets the flag to true, while the ADC.dataReady() method sets it back to false. Sending the sensor reading to the next-hop neighbor via wireless communication is also a long operation. To make sure that it does not block the processing of the ADC.dataReady() event, a separate task is posted to the scheduler. A task is a method defined using the task keyword. In order to simplify the data structures inside the scheduler, a task cannot have arguments.

Figure 8.11 A section of the implementation of SenseAndSend, illustrating the handling of concurrency in nesC.

Thus the sensor reading to be sent is put into a sensorReading variable. There is one source of race condition in the SenseAndSend, which is the updating of the busy flag. To prevent some state from being updated by both scheduled tasks and event-triggered interrupt handlers, nesC provides language facilities to limit the race conditions among these operations.

In nesC, code can be classified into two types:

• Asynchronous code (AC): Code that is reachable from at least one interrupt handler.

• Synchronous code (SC): Code that is only reachable from tasks. Because the execution of TinyOS tasks are nonpreemptive and interrupt handlers preempts tasks, SC is always atomic with respect to other SCs. 	However, any update to shared state from AC, or from SC that is also updated from AC, is a potential race condition. To reinstate atomicity of updating shared state, nesC provides a keyword atomic to indicate that the execution of a block of statements should not be preempted. This construction can be efficiently implemented by turning off hardware interrupts. To prevent blocking the interrupts for too long and affecting the responsiveness of the node, nesC does not allow method calls in atomic blocks. In fact, nesC has a compiler rule to enforce the accessing of shared variables to maintain the race-free condition.

If a variable x is accessed by AC, then any access of x outside of an atomic statement is a compile-time error. This rule may be too rigid in reality. When a programmer knows for sure that a data race is not going to occur, or does not care if it occurs, then a norace declaration of the variable can prevent the compiler from checking the race condition on that variable. Thus, to correctly handle concurrency, nesC programmers need to have a clear idea of what is synchronous code and what is asynchronous code. However, since the semantics is hidden away in the layered structure of TinyOS, it is sometimes not obvious to the programmers where to add atomic blocks.

8.3.3 Dataflow-Style Language: TinyGALS

Dataflow languages are intuitive for expressing computation on interrelated data units by specifying data dependencies among them. A dataflow program has a set of processing units called actors. Actors have ports to receive and produce data, and the directional connections among ports are FIFO queues that mediate the flow of data. Actors in dataflow languages intrinsically capture concurrency in a system, and the FIFO queues give a structured way of decoupling their executions. The execution of an actor is triggered when there are enough input data at the input ports. Asynchronous event-driven execution can be viewed as a special case of dataflow models, where each actor is triggered by every incoming event. The globally asynchronous and locally synchronous (GALS) mechanism is a way of building event-triggered concurrent execution from thread-unsafe components. TinyGALS is such a language for TinyOS.

One of the key factors that affects component reusability in embedded software is the component composability, especially concurrent composability. In general, when developing a component, a programmer may not anticipate all possible scenarios in which the component may be used. Implementing all access to variables as atomic blocks incurs too much overhead. At the other extreme, making all variable access unprotected is easy for coding but certainly introduces bugs in concurrent composition. TinyGALS addresses concurrency concerns at the system level, rather than at the component level as in nesC. Reactions to concurrent events are managed by a dataflow-style FIFO queue communication.

TinyGALS Programming Model

TinyGALS supports all TinyOS components, including its interfaces and module implementations. All method calls in a component interface are synchronous method calls—that is, the thread of control enters immediately into the callee component from the caller component.

An application in TinyGALS is built in two steps:

(1) constructing asynchronous actors from synchronous components, and

(2) constructing an application by connecting the asynchronous components though FIFO queues. An actor in TinyGALS has a set of input ports, a set of output ports, and a set of connected TinyOS components.

An actor is constructed by connecting synchronous method calls among TinyOS components. For example, Figure 8.12 shows a construction of TimerActor from two TinyOS components (i.e., nesC modules), Timer and Clock. Figure 8.13 is the corresponding TinyGALS code. An actor can expose one or more initialization methods. These methods are called by the TinyGALS run time before the start of an application. Initialization methods are called in a nondeterministic order, so their implementations should not have any cross-component dependencies.

Figure 8.12 Construction of a TimerActor from a Timer component and a Clock component.

At the application level, the asynchronous communication of actors is mediated using FIFO queues. Each connection can be parameterized by a queue size. In the current implementation of TinyGALS, events are discarded when the queue is full. However, other mechanisms such as discarding the oldest event can be used. Figure 8.14 shows a TinyGALS composition of timing, sensing, and sending part of the FieldMonitor application in Figure 8.5.

Figure 8.13 Implementation of the TimerActor in TinyGALS.

Figure 8.14 Triggering, sensing, and sending actors of the FieldMonitor in TinyGALS.

Figure 8.15 Implementation of the FieldMonitor in TinyGALS.

Figure 8.15 is the TinyGALS specification of the configuration in Figure 8.14. We omit the details of the SenseAndSend actor and the Comm actor, whose ports are shown in Figure 7.14. The symbol => represents a FIFO queue connecting input ports and output ports. The integer at the end of the line specifies the queue size. The command START@ indicates that the TinyGALS run time puts an initial event into the corresponding port after all initialization is finished.

In our example, an event inserted into the timerStart port starts the HWClock, and the rest of the execution is driven by clock interrupt events. The TinyGALS programming model has the advantage that actors become decoupled through message passing and are easy to develop independently. However, each message passed will trigger the scheduler and activate a receiving actor, which may quickly become inefficient if there is a global state that must be shared among multiple actors. TinyGUYS (Guarded Yet Synchronous) variables are a mechanism for sharing global state, allowing quick access but with protected modification of the data.

In the TinyGUYS mechanism, global variables are guarded. Actors may read the global variables synchronously (without delay). However, writes to the variables are asynchronous in the sense that all writes are buffered. The buffer is of size one, so the last actor that writes to a variable wins. TinyGUYS variables are updated by the scheduler only when it is safe (e.g., after one module finishes and before the scheduler triggers the next module). TinyGUYS have global names defined at the application level which are mapped to the parameters of each actor and are further mapped to the external variables of the components that use these variables. The external variables are accessed within a component by using special keywords: PARAM_GET and PARAM_PUT. The code generator produces thread-safe implementation of these methods using locking mechanisms, such as turning off interrupts.

TinyGALS Code Generation

TinyGALS takes a generative approach to mapping high-level constructs such as FIFO queues and actors into executables on Berkeley motes. Given the highly structured architecture of TinyGALS applications, efficient scheduling and event handling code can be automatically generated to free software developers from writing error-prone concurrency control code. The rest of this section discusses a code generation tool that is implemented based on TinyOS v0.6.1 for Berkeley motes. Given the definitions for the components, actors, and application, the code generator automatically generates all of the necessary code for (1) component links and actor connections, (2) application initialization and start of execution, (3) communication among actors, and (4) global variable reads and writes.

Similar to how TinyOS deals with connected method calls among components, the TinyGALS code generator generates a set of aliases for each synchronous method call. The code generator also creates a system-level initialization function called app_init(), which contains calls to the init() method of each actor in the system. The app_init() function is one of the first functions called by the TinyGALS run-time scheduler before executing the application. An application start function app_start() is created based on the @start annotation. This function triggers the input port of the actor defined as the application starting point. The code generator automatically generates a set of scheduler data structures and functions for each asynchronous connection between actors. For each input port of an actor, the code generator generates a queue of length n, where n is specified in the application definition. The width of the queue depends on the number of arguments of the method connected to the port. If there are no arguments, then as an optimization, no queue is generated for the port (but space is still reserved for events in the scheduler event queue).

For each output port of an actor, the code generator generates a function that has the same name as the output port. This function is called whenever a method of a component wishes to write to an output port. The type signature of the output port function matches that of the method that connects to the port. For each input port connected to the output port, a put() function is generated which handles the actual copying of data to the input port queue. The output port function calls the input port’s put() function for each connected input port. The put() function adds the port identifier to the scheduler event queue so that the scheduler will activate the actor at a later time.

For each connection between a component method and an actor input port, a function is generated with a name formed from the name of the input port and the name of the component method. When the scheduler activates an actor via an input port, it first calls this generated function to remove data from the input port queue and then passes it to the component method. For each TinyGUYS variable declared in the application definition, a pair of data structures and a pair of access functions are generated.

The pair of data structures consists of a data storage location of the type specified in the module definition that uses the global variable, along with a buffer for the storage location. The pair of access functions consists of a PARAM_GET() function that returns the value of the global variable, and a PARAM_PUT() function that stores a new value for the variable in the variable’s buffer. A generated flag indicates whether the scheduler needs to update the variables by copying data from the buffer.

Since most of the data structures in the TinyGALS run-time scheduler are generated, the scheduler does not need to worry about handling different data types and the conversion among them. What is left in the run-time scheduler is merely event-queuing and function triggering mechanisms. As a result, the TinyGALS run-time scheduler is very lightweight. The scheduler itself takes 112 bytes of memory, comparable with the original 86-byte TinyOS v0.6.1 scheduler.

8.4 Node-Level Simulators

Node-level design methodologies are usually associated with simulators that simulate the behavior of a sensor network on a per-node basis. Using simulation, designers can quickly study the performance (in terms of timing, power, bandwidth, and scalability) of potential algorithms without implementing them on actual hardware and dealing with the vagaries of actual physical phenomena. A node-level simulator typically has the following components:

• Sensor node model: A node in a simulator acts as a software execution platform, a sensor host, as well as a communication terminal. In order for designers to focus on the application-level code, a node model typically provides or simulates a communication protocol stack, sensor behaviors (e.g., sensing noise), and operating system services. If the nodes are mobile, then the positions and motion properties of the nodes need to be modeled. If energy characteristics are part of the design considerations, then the power consumption of the nodes needs to be modeled.

• Communication model: Depending on the details of modeling, communication may be captured at different layers. The most elaborate simulators model the communication media at the physical layer, simulating the RF propagation delay and collision of simultaneous transmissions. Alternately, the communication may be simulated at the MAC layer or network layer, using, for example, stochastic processes to represent low-level behaviors.

• Physical environment model: A key element of the environment within which a sensor network operates is the physical phenomenon of interest. The environment can also be simulated at various levels of detail. For example, a moving object in the physical world may be abstracted into a point signal source. The motion of the point signal source may be modeled by differential equations or interpolated from a trajectory profile. If the sensor network is passive—that is, it does not impact the behavior of the environment—then the environment can be simulated separately or can even be stored in data files for sensor nodes to read in. If, in addition to sensing, the network also performs actions that influence the behavior of the environment, then a more tightly integrated simulation mechanism is required.

• Statistics and visualization: The simulation results need to be collected for analysis. Since the goal of a simulation is typically to derive global properties from the execution of individual nodes, visualizing global behaviors is extremely important. An ideal visualization tool should allow users to easily observe on demand the spatial distribution and mobility of the nodes, the connectivity among nodes, link qualities, end-to-end communication routes and delays, phenomena and their spatio-temporal dynamics, sensor readings on each node, sensor node states, and node lifetime parameters (e.g., battery power).

A sensor network simulator simulates the behavior of a subset of the sensor nodes with respect to time. Depending on how the time is advanced in the simulation, there are two types of execution models:

cycle-driven simulation and discrete-event simulation. A cycle-driven (CD) simulation discretizes the continuous notion of real time into (typically regularly spaced) ticks and simulates the system behavior at these ticks. At each tick, the physical phenomena are first simulated, and then all nodes are checked to see if they have anything to sense, process, or communicate. Sensing and computation are assumed to be finished before the next tick. Sending a packet is also assumed to be completed by then.

However, the packet will not be available for the destination node until the next tick. This split-phase communication is a key mechanism to reduce cyclic dependencies that may occur in cycle-driven simulations. That is, there should be no two components, such that one of them computes yk = f (xk) and the other computes xk = g(yk), for the same tick index k. In fact, one of the most subtle issues in designing a CD simulator is how to detect and deal with cyclic dependencies among nodes or algorithm components.

Most CD simulators do not allow interdependencies within a single tick. Synchronous languages, which are typically used in control system designs rather than sensor network designs, do allow cyclic dependencies. They use a fixed-point semantics to define the behavior of a system at each tick. Unlike cycle-driven simulators, a discrete-event (DE) simulator assumes that the time is continuous and an event may occur at any time. An event is a 2-tuple with a value and a time stamp indicating when the event is supposed to be handled. Components in a DE simulation react to input events and produce output events.

In node-level simulators, a component can be a sensor node and the events can be communication packets; or a component can be a software module within a node and the events can be message passings among these modules. Typically, components are causal, in the sense that if an output event is computed from an input event, then the time stamp of the output event should not be earlier than that of the input event. Noncausal components require the simulators to be able to roll back in time, and, worse, they may not define a deterministic behavior of a system.

A DE simulator typically requires a global event queue. All events passing between nodes or modules are put in the event queue and sorted according to their chronological order. At each iteration of the simulation, the simulator removes the first event (the one with the earliest time stamp) from the queue and triggers the component that reacts to that event. In terms of timing behavior, a DE simulator is more accurate than a CD simulator, and, as a consequence, DE simulators run slower. The overhead of ordering all events and computation, in addition to the values and time stamps of events, usually dominates the computation time. At an early stage of a design when only the asymptotic behaviors rather than timing properties are of concern, CD simulations usually require less complex components and give faster simulations. Partly because of the approximate timing behaviors, which make simulation results less comparable from application to application, there is no general CD simulator that fits all sensor network simulation tasks. We have come across a number of homegrown simulators written in Matlab, Java, and C++. Many of them are developed for particular applications and exploit application-specific assumptions to gain efficiency.

DE simulations are sometimes considered as good as actual implementations, because of their continuous notion of time and discrete notion of events. There are several open-source or commercial simulators available. One class of these simulators comprises extensions of classical network simulators, such as ns-2,6 J-Sim (previously known as JavaSim),7 and GloMoSim/QualNet.8 The focus of these simulators is on network modeling, protocols stacks, and simulation performance. Another class of simulators, sometimes called softwarein-the-loop simulators, incorporate the actual node software into the simulation. For this reason, they are typically attached to particular hardware platforms and are less portable. Examples include TOSSIM for Berkeley motes and Em* (pronounced em star) for Linux-based nodes such as Sensoria WINS NG platforms.

8.4.1 The ns-2 Simulator and its Sensor Network Extensions

The simulator ns-2 is an open-source network simulator that was originally designed for wired, IP networks. Extensions have been made to simulate wireless/mobile networks (e.g., 802.11 MAC and TDMA MAC) and more recently sensor networks. While the original ns-2 only supports logical addresses for each node, the wireless/mobile extension of it introduces the notion of node locations and a simple wireless channel model. This is not a trivial extension, since once the nodes move, the simulator needs to check for each physical layer event whether the destination node is within the communication range. For a large network, this significantly slows down the simulation speed.

There are at least two efforts to extend ns-2 to simulate sensor networks: SensorSim from UCLA9 and the NRL sensor network extension from the Navy Research Laboratory.10 SensorSim aims at providing an energy model for sensor nodes and communication, so that power properties can be simulated. SensorSim also supports hybrid simulation, where some real sensor nodes, running real applications, can be executed together with a simulation. The NRL sensor network extension provides a flexible way of modeling physical phenomena in a discrete event simulator. Physical phenomena are modeled as network nodes which communicate with real nodes through physical layers. Any interesting events are sent to the nodes that can sense them as a form of communication. The receiving nodes simply

have a sensor stack parallel to the network stack that processes these events.

The main functionality of ns-2 is implemented in C++, while the dynamics of the simulation (e.g., time-dependent application characteristics) is controlled by Tcl scripts. Basic components in ns-2 are the layers in the protocol stack. They implement the handlers interface, indicating that they handle events. Events are communication packets that are passed between consecutive layers within one node, or between the same layers across nodes. The key advantage of ns-2 is its rich libraries of protocols for nearly all network layers and for many routing mechanisms. These protocols are modeled in fair detail, so that they closely resemble the actual protocol implementations. Examples include the following:

• TCP: reno, tahoe, vegas, and SACK implementations

• MAC: 802.3, 802.11, and TDMA

• Ad hoc routing: Destination sequenced distance vector (DSDV) routing, dynamic source routing (DSR), ad hoc on-demand distance vector (AODV) routing, and temporally ordered routing algorithm (TORA)

• Sensor network routing: Directed diffusion, geographical routing (GEAR) and geographical adaptive fidelity (GAF) routing.

8.4.2 The Simulator TOSSIM

TOSSIM is a dedicated simulator for TinyOS applications running on one or more Berkeley motes. The key design decisions on building TOSSIM were to make it scalable to a network of potentially thousands of nodes, and to be able to use the actual software code in the simulation. To achieve these goals, TOSSIM takes a cross-compilation approach that compiles the nesC source code into components in the simulation. The event-driven execution model of TinyOS greatly simplifies the design of TOSSIM. By replacing a few low-level components, such as the A/D conversion (ADC), the system clock, and the radio front end, TOSSIM translates hardware interrupts into discreteevent simulator events. The simulator event queue delivers the interrupts that drive the execution of a node. The upper-layer TinyOS code runs unchanged.

TOSSIM uses a simple but powerful abstraction to model a wireless network. A network is a directed graph, where each vertex is a sensor node and each directed edge has a bit-error rate. Each node has a private piece of state representing what it hears on the radio channel. By setting connections among the vertices in the graph and a biterror rate on each connection, wireless channel characteristics, such as imperfect channels, hidden terminal problems, and asymmetric links, can be easily modeled. Wireless transmissions are simulated at the bit level. If a bit error occurs, the simulator flips the bit.

TOSSIM has a visualization package called TinyViz, which is a Java application that can connect to TOSSIM simulations. TinyViz also provides mechanisms to control a running simulation by, for example, modifying ADC readings, changing channel properties, and injecting packets. TinyViz is designed as a communication service that interacts with the TOSSIM event queue. The exact visual interface takes the form of plug-ins that can interpret TOSSIM events. Beside the default visual interfaces, users can add application-specific ones easily.

8.5 Programming Beyond Individual Nodes: State-Centric Programming

Many sensor network applications, such as target tracking, are not simply generic distributed programs over an ad hoc network of energy-constrained nodes. Deeply rooted in these applications is the notion of states of physical phenomena and models of their evolution over space and time. Some of these states may be represented on a small number of nodes and evolve over time, while others may be represented over a large and spatially distributed number of nodes, as in tracking a temperature contour.

A distinctive property of physical states, such as location, shape, and motion of objects, is their continuity in space and time. Their sensing and control is typically done through sequential state updates. System theories, the basis for most signal and information processing algorithms, provide abstractions for state update, such as:

where x is the state of a system, u are the inputs, y are the outputs, k is an integer update index over space and/or time, f is the state update function, and g is the output or observation function. This formulation is broad enough to capture a wide variety of algorithms in sensor fusion, signal processing, and control (e.g., Kalman filtering, Bayesian estimation, system identification, feedback control laws, and finite-state automata). However, in distributed real-time embedded systems such as sensor networks, the formulation is not so clean as represented in those equations. The relationships among subsystems can be highly complex and dynamic over space and time. The following concerns, not explicitly raised (8.1) and (8.2), must be properly addressed during the design to ensure the correctness and efficiency of the resulting systems.

• Where are the state variables stored?

• Where do the inputs come from?

• Where do the outputs go?

• Where are the functions f and g evaluated?

• How long does the acquisition of inputs take?

• Are the inputs in uk collected synchronously?

• Do the inputs arrive in the correct order through communication?

• What is the time duration between indices k and k + 1? Is it a constant?

These issues, addressing where and when, rather than how, to perform sensing, computation, and communication, play a central role in the overall system performance. However, these “nonfunctional” aspects of computation, related to concurrency, responsiveness, networking, and resource management, are not well supported by traditional programming models and languages. State-centric programming aims at providing design methodologies and frameworks that give meaningful abstractions for these issues, so that system designers can continue to write algorithms like (8.1) and (8.2) on top of an intuitive understanding of where and when the operations are performed. This section introduces one such abstraction, namely, collaboration groups.

8.5.1 Collaboration Groups

A collaboration group is a set of entities that contribute to a state update. These entities can be physical sensor nodes, or they can be more abstract system components such as virtual sensors or mobile agents hopping among sensors. In this context, they are all referred to as agents. Intuitively, a collaboration group provides two abstractions: its scope to encapsulate network topologies and its structure to encapsulate communication protocols. The scope of a group defines the membership of the nodes with respect to the group. For the discussion of collaboration groups in this chapter, we broaden the notion of nodes to include both physical sensor nodes and virtual sensor nodes that may not be attached to any physical sensor.

In this broader sense of node, a software agent that hops among the sensor nodes to track a target is a virtual node. Limiting the scope of a group to a subset of the entire space of all agents improves scalability. The scope of a group can be specified existentially or by a membership function (e.g., all nodes in a geometric extent, all nodes within a certain number of hops from an anchor node, or all nodes that are “close enough” to a temperature contour). Grouping nodes according to some physical attributes rather than node addresses is an important and distinguishing characteristic of sensor networks.

The structure of a group defines the “roles” each member plays in the group, and thus the flow of data. Are all members in the group equal peers? Is there a “leader” member in the group that consumes data? Do members in the group form a tree with parent and children relations? For example, a group may have a leader node that collects certain sensor readings from all followers. By mapping the leader and the followers onto concrete sensor nodes, we effectively define the flow of data from the hosts of followers to the host of the leader.

The notion of roles also shields programmers from addressing individual nodes either by name or address. Furthermore, having multiple members with the same role provides some degree of redundancy and improves robustness of the application in the presence of node and link failures. Formally, a group is a 4-tuple: G = (A, L, p, R)

Where A is a set of agents; L is a set of labels, called roles; p : A → L is a function that assigns each agent a role; R ≤ L × L are the connectivity relations among roles.

Given the relations among roles, a group can induce a lower-level connectivity relation E among the agents, so that Va, b€A, if ( p(a), p(b))€R, then (a, b) € E. For example, under this formulation, the leader-follower structure defines two roles, L = {leader, follower}, and a connectivity relation, R = {(follower, leader)}, meaning that the follower sends data to the leader. Then, by specifying one leader agent and multiple follower agents within a geographical region (i.e., specifying a map p from a set of agents in A to labels in L), we have effectively specified that all followers send data to the leader without addressing the followers individually.

At run time, the scope and structural dynamics of groups are managed by group management protocols, which are highly dependent on the types of groups. A detailed specification of group management protocols is beyond the scope of this section. Some examples of these protocols are discussed here at a high level.

Examples of Groups

Combinations of scopes and structures create patterns of groups that may be highly reusable from application to application. Here, we give several examples of groups, though by no means is it a complete list. The goal is to illustrate the wide variety of the kinds of groups, and the importance of mixing and matching them in applications.

Geographically Constrained Group. A geographically constrained group (GCG) consists of members within a prespecified geographical extent. Since physical signals, especially the ones from point targets, may propagate only to a limited extent in an environment, this kind of group naturally represents all the sensor nodes that can possibly “sense” a phenomenon. There are many ways to specify the geographic shape, such as circles, polygons, and their unions and intersections. A GCG can be easily established by geographically constrained flooding. Protocols such as Geocasting, GEAR, and Mobicast may be used to support the communication among members even in the presence of communication “holes” in the region. A GCG may have a leader, which fuses information from all other members in the group.

N-hop Neighborhood Group. When the communication topology is more important than the geographical extent, hop counts are useful to constrain group membership. An n-hop neighborhood group (n-HNG) has an anchor node and defines that all nodes within n communication hops are members of the group. Since it uses hop counts rather than Euclidean distances, local broadcasting can be used to determine the scope. Usually, the anchor node is the leader of the group, and the group may have a tree structure with the leader as the root to optimize for communication. If the leader’s behavior can be decomposed into suboperations running on each node, then the tree structure also provides a platform for distributing the computation.

There are several useful special cases for n-HNG. For example, 0-HNG contains only the anchor node itself, 1-HNG comprises the one-hop neighbors of the anchor node, and ∞-HNG contains all the nodes reachable from the root. From this point of view, TinyDB is built on a ∞-HNG group.

Publish/Subscribe Group. A group may also be defined more dynamically, by all entities that can provide certain data or services, or that can satisfy certain predicates over their observations or internal states. A publish/subscribe group (PSG) comprises consumers expressing interest in specific types of data or services and producers that provide those data or services. Communication among members of a PSG may be established via rendezvous points, directory servers, or network protocols such as directed diffusion.

Acquaintance Group. An even more dynamic kind of group is the acquaintance group (AG), where a member belongs to the group because it was “invited” by another member in the group. The relationships among the members may not depend on any physical properties at the current time but may be purely logical and historical. A member may also quit the group without requiring permission from any other member. An AG may have a leader, serving as the rendezvous point. When the leader is also fixed on a node or in a region, GPSR, ad hoc routing trees, or directed diffusion types of protocols may facilitate the communication between the leader and the other members. An obvious use of this group is to monitor and control mobile agents from a base station. When all members in the group are mobile, there is no leader member, and any member may wish to communicate to one or more other members, the maintenance of connectivity among the group members can be nontrivial. The roaming hub (RoamHBA) protocol is an example of maintaining connectivity among mobile agents.

Using Multiple Types of Groups

Mixing and matching groups is a powerful technique for tackling system complexity by making algorithms much more scalable and resource efficient without sacrificing conceptual clarity. One may use

highly tuned communication protocols for specific groups to reduce latency and energy costs. There are various ways to compose groups. They can be composed in parallel to provide different types of input for a single computational entity. For example, one may use a GCG to gather sensor measurements, while using a 1-HNG to select the potential next leader. Groups may also be composed hierarchically, such that a group (or its representative member) is contained by another group. For example, while using multiple groups to compute target trajectories, all tracking leaders of various targets may form a PSG with a base station to report the tracking result to.

8.5.2 PIECES: A State-Centric Design Framework

PIECES (Programming and Interaction Environment for Collaborative Embedded Systems) is a software framework that implements the methodology of state-centric programming over collaboration groups to support the modeling, simulation, and design of sensor network applications. It is implemented in a mixed Java-Matlab environment.

Principals and Port Agents

PIECES comprises principals and port agents. Figure 8.16 shows the basic relations among principals and port agents. A principal is the key component for maintaining a piece of state. Typically, a principal maintains state corresponding to certain aspects of the physical phenomenon of interest.11 The role of a principal is to update its state from time to time, a computation corresponding to evaluating function f in (8.1). A principal also accepts other principals’ queries of certain views on its own state, a computation corresponding to evaluating function g in (8.2).

To update its portion of the state, a principal may gather information from other principals. To achieve this, a principal creates port agents and attaches them onto itself and onto the other principals. A port agent may be an input, an output, or both. An output port agent is also called an observer, since it computes outputs based on the host principal’s state and sends them to other agents.

Observers may be active or passive. An active observer pushes data autonomously to its destination(s), while a passive observer sends data only when a consumer requests it. A principal typically attaches a set of observers to other principals and creates a local input port agent to receive the information collected by the remote agents. Thus port agents capture communication patterns among principals.

Figure 8.16 Principal and port agents.

The execution of principals and port agents can be either time driven or event-driven, where events may include physical events that are pushed to them (i.e., data-driven) or query events from other principals or agents (i.e., demand-driven). Principals maintain state, reflecting the physical phenomena. These states can be updated, rather than rediscovered, because the underlying physical states are typically continuous in time. How often the principal states need to be updated depends on the dynamics of the phenomena or physical events. The executions of observers, however, reflect the demands of the outputs. If an output is not currently needed, there is no need to compute it. The notion of “state” effectively separates these two execution flows.

To ensure consistency of state update over a distributed computational platform, PIECES requires that a piece of state, say x|s, can only be maintained by exactly one principal. Note that this does not prevent other principals from having local caches of x|s for efficiency and performance reasons; nor does it prevent the other principals from locally updating the values of cached x|s. However, there is only one “master copy” for x|s; all local updates should be treated as “suggestions” to the master copy, and only the principal that owns x|s has the final word on its values. This asymmetric access of variables simplifies the way shared variables are managed.

Principal Groups

Principals can form groups. A principal group gives its members a means to find other relevant principals and attaches port agents to them. A principal may belong to multiple groups. A port agent, however, serving as a proxy for a principal in the group, can only be associated with one group. The creation of groups can be delegated to port agents, especially for leader-based groups. The leader port agent, typically of type input, can be created on a principal, and the port agent can take group scope and structure parameters to find the other principals and create follower port agents on them. Groups can be created dynamically, based on the collaboration needs of principals. For example, when a tracking principal finds that there is more than one target in its sensing region, it may create a classification group to fulfill the need of classifying the targets.

A group may have a limited time span. When certain collaborations are no longer needed, their corresponding groups can be deleted. The structure of a group allows its members to address other principals through their role, rather than their name or logical address. For example, the only interface that a follower port agent in a leader follower structured group needs is to send data to the leader. If the leader moves to another node while a data packet is moving from a follower agent to the leader, the group management protocol should take care of the dangling packet, either delivering it to the leader at the new location or simply discarding it. The group management protocol may be built on top of data-centric routing and storage services such as diffusion routing and GHT.

Mobility

A principal is hosted by a specific network node at any given time. The most primitive type of principal is a sensing principal, which is fixed to a sensor node. A sensing principal maintains a piece of (local) state related to the physical phenomenon, based solely on its own local measurement history. Although a sensing principal is constrained to a physical node, other principals may be implemented as software agents that move from host to host, depending on information utility, performance requirements, time constraints, and resource availability. A principal P may also be attached to another principal Q in the sense that P moves with Q. When a principal moves, it carries its state to the new location and the scope of the group it belongs to may be updated if necessary.

Mobile principals bring additional challenges to maintaining the state. For example, a principal should not move while it is in the middle of updating the state. To ensure this, PIECES imposes the restriction that whenever an agent is triggered, its execution must have reached a quiescent state. Such a trigger is called a responsible trigger. Only at these quiescent states can principals move to other nodes in a well-defined way, carrying a minimum amount of information representing the phenomena.

PIECES Simulator

PIECES provides a mixed-signal simulator that simulates sensor network applications at a high level. The simulator is implemented using a combination of Java and Matlab. An event-driven engine is built in Java to simulate network message passing and agent execution at the collaboration-group level. A continuous-time engine is built in Matlab to simulate target trajectories, signals and noise, and sensor front ends. The main control flow is in Java, which maintains the global notion of time. The interface between Java and Matlab also makes it possible to implement functional algorithms such as signal processing and sensor fusion in Matlab, while leaving their execution control in Java. A three-tier distributed architecture is designed through Java registrar and RMI interfaces, so that the execution in  Java and Matlab can be separately interrupted and debugged.

Like most network simulators such as ns-2, the PIECES simulator maintains a global event queue and triggers computational entities—principals, port agents, and groups—via timed events. However, unlike network simulators that aim to accurately simulate network behavior at the packet level, the PIECES simulator verifies CSIP algorithms in a networked execution environment at the collaboration-group level. Although groups must have distributed implementations in real deployments, they are centralized objects in the simulator. They can internally make use of instant access to any member of any role, although these services are not available to either principals or port agents.

This relieves the burden of having to develop, optimize, and test the communication protocols concurrently with the CSIP algorithms. The communication delay is estimated based on the locations of sender and receiver and the group management protocol being used. For example, if an output port of a sensing principal calls sendToLeader (message) on its container group, then the group determines the sensor nodes that host the sensing principal and the destination principal, computes the number of hops between the two nodes specified by the group management protocol, and generates a corresponding delay and a bit error based on the number of hops. A detailed example of using this simulator is given in the next section.

8.5.3 Multitarget Tracking Problem Revisited

Using the state-centric model, programmers decouple a global state into a set of independently maintained pieces, each of which is assigned a principal. To update the state, principals may look for inputs from other principals, with sensing principals supporting the lowest-level sensing and estimation tasks. Communication patterns are specified by defining collaboration groups over principals and assigning corresponding roles for each principal through port agents.

A mobile principal may define a utility function, to be evaluated at candidate sensor nodes, and then move to the best next location, all in a way transparent to the application developer. Developers can focus on implementing the state update functions as if they are writing centralized programs. To make these concepts concrete, let us revisit the multitarget tracking system introduced in Chapter 2. Recall that in Figure 2.5, the tracking of two crossing targets can be decomposed into three phases:

1. When the targets are far apart, the tracking problem can be treated as a set of single-target tracking sub problems.

2. When the targets are in proximity of each other, they are tracked jointly due to signal mixing.

3. After the targets move apart, the tracking problem becomes two single-target tracking sub problems again.

To summarize, there are two kinds of target information that the user cares about in this context: target positions and target identities.

In the third phase above, in addition to the problem of updating track locations, there is a need to sort out ambiguity regarding which track corresponds to which target. We refer to this problem as the identity management problem. Specifically, one must keep track of how the identities mix when targets cross over, and update identity information at the other node when credible target identity evidence is available to one node.

The identity information may be obtained by a local classifier or by an identity management protocol across tracks. In PIECES, the system is designed as a set of communicating target trackers (MTTrackers), where each tracker maintains the trajectory and identity information about a target or a set of spatially adjacent targets. An MTTracker is implemented by three principals: a tracking principal, a classification principal, and an identity management principal, as shown in Figure 8.17. In the first phase, the identity state of the track is trivial; thus no classification and identity management principals are needed.

Figure 8.17 The distributed multi-object tracking algorithm as implemented in the state-centric programming model, using distributed principals and agents as discussed in the text. Notice that the state-centric model allows an application developer to focus on key pieces of state information the sensor network creates and maintains, thus raising the abstraction level of programming

A tracking principal updates the track position state periodically. It collects local individual position estimates from sensors close to the target by a GCG with a leader-follower relation. The tracking principal is the leader, and all sensing principals within a certain geographical extent centered about the current target position estimate are the followers. The tracking principal also makes hopping decisions based on its current position estimate and the node characteristic information collected from its one-hop neighbors via a 1-HNG. When the principal is initialized, it creates the agents and corresponding groups. Behind the scene, the groups create follower agents with specific types of output, indicated by the sensor modalities. Without further instructions from the programmer, the followers periodically report their outputs to the input port agents.

Whenever the leader principal is activated by a time trigger, it updates the target position using the newly received data from the followers and selects the next hosting node based on neighbor node characteristics. Both the classification principal and the identity management principal operate on the identity state, with the identity management principal maintaining the “master copy” of the state. In fact, the classification principal is created only when there is a need for classifying targets. The classification principal uses a GCG to collect class feature information from nearby sensing principals in the same way that tracking principals collect location estimates. The identity management principal forms an AG with all other identity management principals that may have relevant identity information. They become members of a particular identity group only when targets intersect and their identities mix. Both classification principals and identity management principals are attached to the tracking principal for their mobility decisions. However, the formation of an AG among these three principals also provides the flexibility that they can make their own hopping decisions without changing their interaction interface.

Simulation Results

Figure 8.18 shows the progression of tracking two crossing targets. Initially, when the targets are well separated, as in Figure 8.18(a), each target is tracked by a tracker whose sensing group is pictured as

a shaded disk.

Figure 8.18 Simulation snapshots: Sensor nodes are indicated by small circles, and the crossing lines indicate the true trajectories of the two targets. One geographically constrained group is created for each target. When the two targets cross over, their groups merge into one.

The hosting node of the tracking principal is plotted in solid white dots, and the hosts for corresponding sensing principals are plotted in small, empty white circles inside the shaded disks. Since the targets are well separated, each identity group contains only one member—the identity management principal of a tracker. As the targets move toward the center of the sensor field, the sensing groups move with their respective track positions. In Figure 8.18(b), the two separate tracking groups have merged. A joint tracking principal updates tracks for both targets. The reason for the merge is that when the two targets approach each other, it is more accurate to track the targets jointly, rather than independently, due to the effect of signal mixing. Finally, as the targets move away from each other, the merged tracking group splits into two separate single target tracking groups that proceed to track each target separately, as shown in Figure 8.18(c). At this point, the identities of the targets are mixed, so that an identity group is created to contain the two identity management principals from both trackers.

8.6 RECENT ADVANCES IN WIRELESS NETWORKS

8.6.1. INTRODUCTION

It is expected that by the end of this decade, the predominant mode of Internet access will be over wireless networks. In order to support high data rates on short ranges, new promising technologies such as the ultra-wide band (UWB) transmission scheme and optical wireless wavelength division multiplexing (WDM) networks are currently under research. In addition to the demands on increasing the performance of the lower layers of the system, pressure is on to efficiently reuse the existing frequencies, which also demands allocation of newer bands for commercial wireless communications. The additional areas of interest in next-generation wireless networks include freedom in selection of a particular network from a set of heterogeneous wireless networks, seamless handoff across such networks, support for multimedia traffic, and all the above services at an affordable cost.

The first step in this is the development of wireless fidelity (Wi-Fi) systems that integrate the high-speed wireless LANs with the wide-area packet cellular infrastructure. The demand for high bandwidth for high-quality multimedia traffic has also resulted in utilizing the WDM technology in wireless communication. This chapter discusses the recent advances in the area of wireless networking such as UWB technology, wireless fidelity, optical wireless communication, and IEEE 802.11a/b/g.

8.7 ULTRA-WIDE-BAND RADIO COMMUNICATION

The major differences between the ultra-wide band (UWB) technology and the existing narrow-band and wide-band technologies are the following: (i) The bandwidth of UWB systems, as defined by the Federal Communications Commission (FCC), is more than 25% of the center frequency or a bandwidth greater than 500 MHz. (ii) The narrow-band and wide-band technologies make use of a radio frequency (RF) carrier to shift the base band signal to the center of the carrier frequency, whereas theUWB systems are implemented in a carrier less fashion in which the modulation scheme can directly modulate base band signals into an impulse with very sharp rise and fall time, thus resulting in a waveform ranging several GHz of bandwidth.

These impulses have a very low duty cycle.1 A simple example of UWB transmission is any radio frequency (RF) transmission with a bandwidth of 500 MHz at a center frequency of 2GHz. The IEEE 802.11b is a narrow-band system with a 22 MHz bandwidth with center frequencies ranging from 2.412 GHz to 2.462 GHz. One of the major approaches for generating a UWB waveform is the impulse-based approach which is explored later in this section. The principles behind the operation of UWB technology had been applied in radar applications since the 1980s. With proper emission restrictions (restrictions on the upper limit on the transmission power) in place, the UWB spectrum can overlay the existing narrow-band spectrum, resulting in much more efficient use of the existing radio spectrum.

Figure 8.19 illustrates the upper limits of transmission power permitted for the UWB system in comparison with the IEEE 802.11a. The 802.11a standard has three bands, each with 100 MHz bandwidth with an upper limit of effective isotropically radiated power (EIRP),2 as follows: 16 dBm for the band 5.15-5.25 GHz, 24 dBm for the band 5.25-5.35 GHz, and 30 dBm for the upper band with frequency range 5.725-5.825 GHz. This results in a power spectral density of -3 dBm/MHz, 4 dBm/MHz, and 10 dBm/MHz, respectively. In contrast to these, FCC has restricted the maximum power emitted by the UWB systems to be a power spectral density of -41.3 dBm/MHz. As depicted inFigure 14.1, the UWB signals appear as noise to the legacy systems that utilize the same spectrum.

Figure 8.19 Spectrum of UWB systems compared with IEEE 802.11b and 802.11a.

8.7.1 Operation of UWB Systems

The operation of UWB systems is based on transmission of ultra-short pulses (that yields a wide-band bandwidth signal) which are also called monocycles. Each monocycle is similar to a single cycle of an ultra-high frequency sine wave and is a single ultra-short pulse. The pulse widths of monocycles range from 0.10 to 1.6 ns with pulse-to-pulse intervals of between 25 and 1,000 ns. A single monocycle of width 0.5 ns is shown in Figure 8.20(a). Figure 8.20 (b) shows a pulse train involving a sequence of monocycles generated at regular intervals. Monocycles are inherently wide bandwidth signals. The time domain function of a Gaussian monocycle (any sharp pulse whose time domain behavior can be represented as the first derivative of a Gaussian distribution) can be represented as , where t is time and τ is the time-decaying factor that decides the duration of the monocycle. It can be represented as in the frequency domain, where the center frequency (fc) is proportional to . This means that the shorter the monopulse duration, the higher the center frequency. For a monopulse similar to the one shown in Figure 8.20 with a pulse width of 0.2 ns, the center frequency is around 5 GHz. The frequency domain function of an ultra-short pulse shows that the frequency components spread across (bandwidth) a wide range greater than 110% that of the center frequency. For a monopulse with 0.2 ns duration, the bandwidth is approximately around 5 GHz, as illustrated in Figure 8.21.

Figure 8.20. An illustration of a monocycle and pulse train.

Figure 8.21. Bandwidth of a monocycle.

In the pulse train of monocycles, the information is modulated either by using pulse position modulation (PPM) or similar modulation techniques. The effect of modulation with PPM on a train of monocycles is illustrated in Figure 8.22 (a), in which the precise timing of the monocycle varies with respect to its nominal position. For example, in a 100 Mpps (million pulses per second) system, monocycles are transmitted every 10 ns (refer to Figure 8.22). In this case, a PPM modulation may advance the monocycle for 10 picoseconds for representing a digital bit "1" and delay the monocycle for 10 picoseconds representing a digital "0". An encoding can be performed over the unmodulated pulse train to provide distinct time-hopping codes for channelizing the pulse train (channelizing refers to provisioning of multiple channels in the same spectrum).

In such a multiple access system, each user would have a unique pseudo-random noise (PN) codes for encoding the pulse trains so that multiple simultaneous transmissions can coexist. Figure 8.22 (b) shows the PN coded pulse train without data content. The nominal pulse position in the train is decided by the PNcode as shown in Figure 8.22, where the pulses appear with different time differences from the reference points Tn+1, Tn+2, and Tn+3. This encoding process on the modulated pulse train makes the signal appear like white noise (noise containing all spectral components). A receiver cannot detect the transmission and the corresponding data without having the unique pseudorandom (time-hopping) code with which it was coded, even when the receiver is located at very close proximity to the transmitter.

Figure 8.22. The PPM coded pulse train.

8.7.2 A Comparison of UWB with Other Technologies

The trends that drive the short-range wireless technologies, including UWB technology, are the following: (i) The growing demand for data and multimedia capability in portable devices at high data rate but at low cost and power consumption. (ii) Increasing pressure on the wireless spectrum demanding higher reuse. (iii) Decreasing semiconductor cost and availability of low-power devices. Some of the competing technologies for such wireless access scenario are IEEE 802.11b, IEEE 802.11a, and Bluetooth.

The bandwidth, transmission range, and capacity comparison in bps/m2 among 802.11b, Bluetooth, 802.11a, and UWB is illustrated in Figures 8.23, 8.24, and 8.25, respectively. In North America, the 802.11b spectrum ranges from 2,400 MHz to 2,483 MHz and is divided up into 11 channels from 2,412 MHz to 2,462 MHz, spaced 5 MHz apart, as illustrated in Figure 8.26. However, each channel is 22 MHz wide, resulting in great overlap. For example, the spectrum of channel 1 centered at 2,412 MHz overlaps with neighboring channels 2, 3, 4, and 5. This leads to a situation where, at any given point, only three channels (1,6, and 11) can be simultaneously used. In Europe, of the 13 permitted channels, four channels can be used simultaneously.

Figure 8.23. Bandwidth comparison of UWB with other competing technologies.

Figure 8.24. Range comparison of UWB with other competing technologies.

Figure 8.25. Capacity comparison of UWB with other competing technologies.

Figure 8.26. An illustration of 802.11b channels.

Hence in North America, three channels among the 11 channels can operate simultaneously, providing a total bandwidth of 33 Mbps. The capacity of the system with respect to the area of coverage can be 33 Mbps over an area of a circle with a 100 m radius. Hence, 802.11b has a capacity of 1 Kbps/m2. Bluetooth has a transmission range of 10 m in its low-power mode and a peak bandwidth of 1 Mbps. There can be around 10 simultaneous piconets operating in the 10 m radius circle, making the total bandwidth 10 Mbps.

Hence, Bluetooth has a capacity of 31 Kbps/m2. The 802.11a has an operating transmission range of around 50 m with a maximum bandwidth of 54 Mbps, and 12 such systems can simultaneously operate within a 50 m radius circle. Thus the capacity of 802.11a is approximately around 82 Kbps/m2. UWB systems can vary widely in their projected capacities, but systems with 50 Mbps with a transmission range of 10 meters have been demonstrated. With minimal interference, more than six such systems can operate simultaneously. Hence, UWB transmission systems have a capacity of 955 Kbps/m2. The capacity of each technology is shown in Figure 8.25. Thus, UWB systems have great potential for support of future high-capacity short-range wireless systems.

8.7.3 Major Issues in UWB

With the evolution of wireless networks with higher data rates, similar advancement in all the layers of protocol stack is required to make use of the high data rates provided by the physical layers. The increasing application space for multimedia communications puts additional requirements on latency and delay performances. The error-prone and time-varying nature of wireless link results in large delay, delay variations, and misordering of packets. In this section, the major issues at the physical and MAC layers are discussed.

Physical Layer The wide-band receiver used for the UWB system is susceptible to being jammed by the traditional narrow band that operates within the UWB pass band. Issues such as the wide bandwidth required for the filters and antenna subsystems and filter-matching accuracy are major physical-layer issues difficult to solve without adding to the cost of the physical-layer interface. At the receiver, accurate timing is required for detecting the modulated narrow pulses accurately (refer to Figure 8.22). In addition to all the above, noise from the on-board micro-controller can also result in interference that cannot be easily solved using traditional mechanisms such as band-pass filters, due to the wide bandwidth of the system.

MAC Layer

The most important issues in the design of MAC-layer protocol for UWB systems are the following: (i) controlling channel access, (ii) maintaining QoS, and (iii) providing security. In addition to these, the design of MAC protocols for UWB systems is dictated by other properties of UWB. For example, UWB systems have unique features such as precise timing or position information. The position information can be obtained by the following feature. A 10 GHz UWB system can distinguish signal echos (signal echos refer to the multipath signals that arrive at the receiver through different paths with corresponding delays) that differ by 100 picoseconds.

By analyzing the different signal echos that take different delays, a nearly exact position and time information can be obtained. For a 10 GHz system, the delay differences could be of the order of 100 ps and hence an accuracy of 3 cm in distance estimation can be achieved. Utilization of this feature at the MAC layer can improve performance of multimedia unicast and multicast communication. MAC protocols can also benefit from the flexibility of trading off the throughput with respect to the transmission range. The peak amplitude of pulses and the pulse repetition frequency (PRF) can be varied to obtain constant average power. Using this mechanism, MAC protocol can provide different data rates and transmission ranges on a per-link or per-packet basis. UWB systems can be implemented using spread spectrum technology in order to provide better coexistence with the existing systems. Above all, another major aspect of the design of MAC protocol for UWB systems is the compatibility and coexistence of UWB systems with existing WPANs and WLANs.

8.7.4 Advantages and Disadvantages of UWB

Promising applications of UWB in communication include cable-free audio/video devices, broadband WPANs, and high-speed wireless links. UWB systems are potential candidates for high data rate, low power, and short- to medium-range communication applications. UWB systems have a wide range of applications other than communications, some of which are automobile collision-detection devices, medical imaging similar to x-rays and ultrasound scans, through-wall imaging for detecting people and objects in law enforcement applications, and ground-penetrating radars in construction applications. The major advantages of UWB systems include simplicity of implementation, high data rate, inherent robustness to multipath fading, flexibility of operation, low power consumption, and low cost of implementation. The disadvantages of UWB systems include stringent design requirements of communication subsystems and chances of interference from the existing technologies.

8.8 WIRELESS FIDELITY SYSTEMS

Wireless fidelity (Wi-Fi) system is the high-speed wireless LAN that was originally intended to extend the wired Ethernet in offices to wireless clients. The coverage area and ability to support high bit rates are the two major reasons behind the name Wi-Fi. Though the popular wireless LAN standards IEEE802.11b and 802.11a are considered as the standard Wi-Fi candidates, conceptually any high-speed wireless LAN protocol such as HiperLAN can be used. The integration of Wi-Fi hotspots (wireless LAN access points) with wide area wireless networking technologies such as GSM and GPRS provides an added advantage for the mobile nodes.

Such an integrated system provides secure, reliable, and high speed wireless connectivity. A Wi-Fi network can be used to connect computers to each other, to the Internet, and to wired networks. Wi-Fi networks operate in the unlicensed 2.4 GHz and 5 GHz radio bands, with an 802.11b or 802.11a, or with products that contain both bands (dual band), so that they can provide an enriched network experience. Wi-Fi systems are potential candidates for provisioning high-speed multimedia content delivery in areas such as indoor offices, airport lounges, and shopping malls. The Wi-Fi Alliance is a nonprofit international association formed in 1999 to certify interoperability of IEEE 802.11-based products in order to make the objectives of Wi-Fi a reality. The advantages of Wi-Fi systems are ease-of-use, high-speed Internet access, low cost of operation, and flexibility of reconfiguration.

8.8.1 The Service Provider Models for Wi-Fi Systems

• The Wi-Fi micro carrier model: In this model, small business operators can set up their own access points (APs) and maintain customer relations and billing with subscribers. An example of this category is a restaurant operating a small Wi-Fi system with a set of APs on its premises.

• The franchisee-franchisor model: This model for Wi-Fi systems is that a franchisor company making an agreement with a franchisee (e.g., a restaurant which has an inbuilt Wi-Fi system for its internal purposes) for providing Wi-Fi connectivity on a revenue-sharing basis. The external communication, access network costs, and back-office softwares may be supplied and maintained by the franchisor. Hence, the franchisor company can extend its services to the public.

• The Wi-Fi carrier model: In this model, a particular company referred to as Wi-Fi carrier can own, deploy, and operate a number of Wi-Fi system-enabled APs at public places. The subscribers can utilize the designated carrier's network services in their coverage area based on acceptable billing models.

• The Wi-Fi aggregator model: This model refers to an abstract service provider which strikes wholesale partnerships with Wi-Fi operators. Such aggregators mainly focus on two major things: (i) reselling of the services provided by the Wi-Fi operators and (ii) giving their subscribers access to a large number of networks. The advantages of this model are easy scale-up of network coverage as the aggregator does not own the infrastructure and hence, by having more partnerships, a service provider can increase the coverage area and the customer base.

• The extended service provider model: The synergistic operation of Wi-Fi systems with existing cellular systems, especially with the 3G systems, can increase profits for cellular operators. This can even lead to reduction in the deployment cost of 3G systems. The widespread deployment of Wi-Fi systems can be considered as complementing the 3G systems. Also, the availability of wireless devices equipped with Wi-Fi and cellular interfaces encourages the possibility of switching to the Wi-Fi systems whenever an AP is detected. Vertical handoff (the handoff performed across two networks which are operating at widely varying coverage regions, for example, the handoff performed between a wireless LAN and wireless WAN) can be used to switch back to the wide area cellular networks as and when necessary in such cases.

Thus, the extended service provider model envisions the provisioning of Wi-Fi services as an extension to the existing service provided by the cellular network operators. An illustration of such a model is shown in Figure 8.27, where the users can access information and data through either the Wi-Fi AP or through the 3G BS. Referring to Figure 8.27, the node C can communicate directly with the BS in order to make a call to the destination node E, and node K can communicate with the BS through the nearest Wi-Fi AP. This choice of selection can be based on network availability, cost of access, bandwidth availability, and specific user requirements. Such a system requires placement of APs at places such as crowded traffic junctions, public places, and railway stations. The presence of multiple service providers trying to use this model would result in managed utilization of spectrum in the bands assigned for 802.11.

8.8.2 Issues in Wi-Fi Systems

The major issues in Wi-Fi systems are the following:

Security

Security in Wi-Fi systems is important and requires viable solutions. The IEEE has proposed a long-term security architecture for 802.11, which is referred to as robust security network (RSN). The major objectives of RSN, which is based on IEEE 802.1x, are access control, authentication, and key management. Even though the research work done by Mishra and Arbaugh in indicates the vulnerability of the initial versions of the IEEE 802.1x, IEEE 802.1x remains the only viable option as of this writing.

Figure 8.27. A typical scenario of a Wi-Fi system.

In this section, the building blocks of security, major elements of threats and major types of attacks that a Wi-Fi system could be subjected to, are discussed. In the infrastructure mode of operation of 802.11 standard, a client has to establish a relation with an AP called an association. There are three main states in the transition process of an association. These are (i) unauthenticated and unassociated, (ii) authenticated and unassociated, and (iii) authenticated and associated.

The major elements in a Wi-Fi system that can contribute to its vulnerability are discussed below.

• The eavesdropper: Wireless medium is inherently broadcast, making it trivial for the adversary with a good receiver to eavesdrop on other stations' traffic. The use of a single key for encryption proved inadequate for this purpose. Dynamic changing of the encryption key is a must for making the eavesdropping a difficult task. Since the majority of the Wi-Fi systems are not WEP (wired equivalent privacy – any privacy solution in which the effort required to break the cipher is expected to be roughly equivalent to the effort required by an intruder to tap into a wired Ethernet)-enabled, it is easier for an inexperienced hacker to snoop on valuable information such as credit card number, PIN numbers, or social security numbers from a public hot spot Wi-Fi system.

• The rogue client: The authenticated client that has intentions of acting against the organization is another problem aggravated by the wireless environment. A Wi-Fi client running a routing protocol and an additional wired interface can easily use its authentication information on its corporate wireless network infrastructure to transfer valuable information. This problem is extremely difficult to trace and prevent as the loss incurred cannot easily be detected.

• The rogue AP: The rogue AP is the unauthorized AP attached to an organization's network, either on its premises or in its authorized employee's home. An AP without the necessary security provisions, or in situations where the security is disabled temporarily, can act as a rogue AP. Even with security features such as WEP, organizations that use virtual private network (VPN) to secure their wireless LANs may be compromising valuable data. A VPN is a communication network between two or more machines or networks, built for the private use of an organization over a shared public infrastructure such as the Internet. The mechanisms to identify rogue APs include sniffing, pinging, and wide spectrum channel monitoring around the premises of the organization.

The two major types that make a Wi-Fi network particularly vulnerable are session hijacking and man-in-the-middle attack. Both exploit the authentication process.

• Session hijack attack: In this case, a hijacker node waits until the successful completion of a node's authentication process with an AP. On completion of the authentication, the hijacker sends a forged disassociate message that appears to be originated by the AP. On reception of the message, the original node gets disassociated. The AP continues to consider the node as part of the network. Now the hijacker, using the originally authenticated node's address, can utilize network services until the session expires.

• Man-in-the-middle attack: When an adversary spoofs messages to represent itself as AP to a node under attack, it can obtain the victim's authentication information. Using this information, the adversary authenticates with another AP masquerading as the victim node. This attack is known as man-in the-middle attack. This attack is particularly effective when the victim node is not present within the transmission range of the AP.

Authentication

The IEEE 802.1x standard is proposed for providing authentication and controlling traffic to a protected network, as well as for dynamically changing encryption keys. The use of the extendible authentication protocol (EAP) as a framework for wired and wireless network provides a mechanism for multiple authentication methods such as certificate-based authentication, Kerberos3 authentication method, token cards, one-time passwords, and public key authentication. The interactions between specific entities in 802.1x is discussed below. The beginning of a series of messages that mark the authentication process is when a client (referred to as the supplicant in 802.1x terminology) sends an EAP-start message to the AP (authenticator). The authenticator responds with an EAP-request identity message. The client replies with an EAP-response packet that contains the identity of authentication server. This authentication server uses a specific authentication algorithm to verify the credentials of the client by performing verification based on any of the multiple authentication mechanisms mentioned above. Upon verification of the client's information, the AP originates an EAP-success or EAP-failure packet to the client. Once the authentication is successful, the AP permits data traffic between the client and the network.

Quality of Service (QoS)

Provisioning of QoS is important in supporting time-sensitive traffic such as voice and video. The IEEE802.11e standard which is under consideration is aiming at providing enhanced QoS for Wi-Fi systems.

Economics of Wi-Fi

• Billing schemes: Billing for Wi-Fi systems assumes importance as the commercial viability is a major factor for the existence of the Wi-Fi systems. The possible billing schemes include the following. The entity that has the responsibility of customer care and billing can be different from the one which actually provides network access service. Such an agency is referred to as a billing agency. The billing agency can employ different methods to accept payments from the customers. The major billing approaches that can be employed in the case of Wi-Fi systems are flat-rate schemes and volume-based pricing.

In flat-rate schemes, the user is permitted to utilize network services for a specified amount of time without restricting the bandwidth. The volume-based approach charges the customer based on the amount of data transacted over the network. In addition to both these schemes, business establishments can provide Wi-Fi services as a value addition to the customers visiting the premises for the core business activity. In such cases, the billing is not considered as the bandwidth provided belongs to the organization's unutilized bandwidth; it can even be considered without additional charging.

• Revenue sharing model: In the franchisee-franchisor model and the aggregator model, the sharing of revenue is important as there exist multiple business entities in the process of customer relationship, billing, and providing service. Different revenue-sharing models that include a fixed fraction-sharing model (in which the amount shared among the parties involved is prefixed) and a variable-fraction volume-based sharing model (in which the percentage of revenue that goes to different parties involved varies with the volume of data transferred) can be employed. Such systems can consider a constant-fraction for the user per bit of data transferred, and in the high-traffic-density environments, a variable rate per bit of data transferred can be used. In the variable-fraction per bit of data transferred, application level mechanisms which communicate the cost of communication at any particular location is essential.

Spectrum Issues

The issues related to spectrum management are important as Wi-Fi becomes popular with its increasing use as a critical business communication infrastructure. The current allocation of free ISM band in the 2.4 GHz band for 802.11b raises several questions of interference. The source of interferences can be either naturally generated by other devices that are designated to operate at the same band or artificially generated by a rogue interference generator node. Since ISM band is unlicensed, any user with an 802.11 interface can disrupt communication at a specific location without inviting prosecution.

The major interference sources for Wi-Fi systems are the following: (i) interference from cordless phones, microwave ovens, and Bluetooth-enabled devices and (ii) interference from jammers.

The first issue can be reduced to some extent by the following ways: (i) use of different frequency-hopping patterns for communication-technology-related interference as used in Bluetooth standard and (ii) proper electromagnetic shielding in devices using microwave band for non communication related purposes such as microwave ovens.

8.9 Future Research Directions

As sensors, actuators, embedded processing, and wireless become ubiquitous, we need new ways to network, organize, query, and program them. In the following, we outline several research directions that address the grand challenge problems for sensor network research.

8.9.1 Secure Embedded Systems

Ensuring security and privacy is one of the highest priorities for sensor network systems. Whether they are designed for commercial applications such as vehicle-to-vehicle safety-related communications or for military battlefield monitoring applications, one must authenticate the source of data to prevent the malicious injection of bogus data. We need to adapt existing public key infrastructure (PKI) for sensor networks or develop new protocols that are lightweight and can be implemented on resource-limited sensor nodes. In some cases, asymmetric protocols may be preferable since base stations or cluster leader nodes have more computational capability and energy supply than other sensing nodes.

For many civilian applications, we must also ensure that access to sensor databases may only be granted to authorized users such as law enforcement agencies. Sometimes it may be necessary to provide a multi-resolution access to sensor data so that users with different levels of clearances have different access to the information. Research in statistical databases has examined similar problems, by introducing controlled “blurring” to data. For example, in an employee database, one may convolve salary data with a zero-mean Gaussian noise so that the average remains the same but individual salary data is distorted to protect privacy rights.

8.9.2 Programming Models and Embedded Operating Systems

In a typical embedded software (EmSoft) application today, half of the code may be dedicated to management of threads, events, messages, or timing issues, and not with the application itself. How can an application developer, without an extensive computer science background in networking and embedded software, easily write software for a network of thousands of embedded devices? Needed is a new breed of embedded operating systems and design environments that provide a mental model closer to the way people think about these applications.

One early example of this is the PIECES framework, which allows an application developer to write code for a sensor network signal processing and tracking application using a state-centric model of programming, a paradigm much closer to the way application engineers reason about the problem. Further research is needed to develop generic software frameworks, a set of APIs for sensor networks, together with a suite of design tools and ad hoc routing libraries, from which an application developer can mix and match to prototype sensor network applications. The software environment should provide the necessary abstractions of the systems so that developers can spend more time designing and optimizing application-level logic than fiddling with low-level thread management.

8.9.3 Management of Collaborative Groups

The ability to form dynamic, ad hoc groups of nodes according to the requirements of a sensing task and resource availability is at the center of many sensor network systems and applications. We will need networking protocols for nodes participating in a group to maintain communications despite mobility or node and link failure. A key consideration in the design is to minimize unnecessary energy and bandwidth usage. Anticipated usage patterns and data rates may determine whether it should be infrastructure-based or infrastructure-less (e.g., using source-initiated ad hoc routing). Once a group is formed, we will need to develop protocols for nodes to join or leave the group, for groups to merge with others, or for a group to split into multiple subgroups. We have described an effective way of forming and managing groups for a tracking application. Another problem is the maintenance of common state information about the nodes in a group—for example, the size of the bounding box for the group. As nodes move, such information should be maintained in a consistent and persistent manner and in a way that can be efficiently located by all nodes in the group.

8.9.4 Lightweight Signal Processing

Lightweight signal processing algorithms refer to methods that require relatively little floating-  point computation and less memory storage than those that are floating-point intensive such as Fast Fourier Transform (FFT). This is attractive, since the signal processing algorithms are targeted for resource-limited nodes such as the Berkeley motes.

Future research is needed to identify the class of signal processing problems that may be efficiently implemented using integer computation. An example of such combinatorial signal processing is the computation of topological features of a signal landscape, such as maxima, minima or saddles. There, signal peaks, which correspond to locations of signal sources, may be recovered using simple peer-to-peer signal-level comparison and propagation. More research will be needed to identify other properties of signal sources that can be likewise recovered, without extensive computation. The field of signal and image processing has developed many algorithms for image restoration and enhancement and for feature extraction and recognition. Some of these algorithms, such as those based on relaxation or wavelet computation, can be effectively decentralized and implemented in sensor networks. An interesting research direction is to recast these algorithms in the context of sensor networks.

8.9.5 Networks of High-Data-Rate Sensors

Though most of the examples we have discussed deal with low data rate sensors, such as acoustic microphones, there is increasing interest in networking high-data-rate sensors as well, such as cameras and other imagers. The sheer volume of data generated by such sensors makes it imperative to extract and transmit high-level descriptions of what is sensed, as opposed to raw signals. Further, the data sensed by nearby sensors is often highly correlated; such redundancy must be detected and removed—here distributed data compression techniques come into play. Unlike the usual compression scenario of, say, broadcasting a video signal, where the compressor can be an expensive algorithm while decompressors must be cheap, here the roles are reversed. Sensor node compressors must be cheap, but we can afford more expensive decompression at a central site. Mixed networks must also be studied, where detections are carried out by low-bandwidth sensors and in turn activate high bandwidth cameras to obtain detailed information about an event of interest.

8.9.6 Google® for the Physical World

Interacting with a distributed sensor system requires research in user interface and search engines that are different from those for IP-based networks. Since the primary purpose of a sensing system is to gather information, we’d like to be able to “browse” and ask high-level questions about the physical environment. Instead of asking moisture sensor #153 for its current reading, why not query the lawn to see if it needs more water? What is needed is an XML-like language for sensor networks, and search techniques for matching higher-level queries with physical signals. In some cases, the search may trigger additional sensing actions. Research in distributed storage, data-centric naming, and information extraction will provide the necessary building blocks. Also needed is research in information visualization. Distributed sensor data collected by a sensor network may be parameterized in a very high dimensional space. We need techniques to project the data to lower-dimensional subspaces and to effectively visualize the data.

8.9.7 Closing the Loop with Actuators

In many application scenarios, the sensors are the “eyes” that inform the “hands” and “legs” what to do. The hands and legs are the actuators. For example, moisture sensors may sense that a patch of green is dry and use the information to command the sprinklers near that spot. Or, sensors may be on wheels, and can move to different locations, thus providing an extra degree of freedom in sensor placement. Common to these problems are the tight coupling between sensing, decision making, and actuation. Studying sensors and actuators in a closed loop brings out many interesting research issues that are not apparent in sensor-only systems. Sensor data in the closed loop systems is typically locally used, demanding some degree of real-time responsiveness. Energy usage is also more demanding, since typically actuators (e.g., motors) draw more electrical current than sensors in order to effect an environment.

This raises the important issue of power management, which now must optimize sensing, processing, and communication together with actuation. Likewise, sensor tasking must be co-designed with actuator tasking, with the overall goal of effecting the external environment in a desired way. At the same time, the sensor-actuator closed loop brings additional benefits to sensor networks. Mobility of nodes can place sensors on demand, maximizing sensor field coverage and connectivity. Mobility could also bring additional power supply to nodes that are nearly depleted of energy, say, using a mobile charging station. Robotic systems may utilize stationary sensors as extra antennas for navigational purposes.

8.9.8 Distributed Information Architecture

Distributed information architectures deal with how information is organized and manipulated in a sensor network. An inference task typically is more complex than finding the maximum or average of sensor values. It may involve reasoning about motion or relations about observed phenomena. A key problem is distributed inference, mapping representations and techniques such as graphical models or dynamic Bayesian networks to sensor networks. Information double counting, as discussed earlier, is a major problem for distributed inference and may be addressed using approximate algorithms. A desirable property of distributed inference is that inference algorithms and architectures should be composable, allowing for scalable computation over distributed data.

Another important problem is the steering of sensing foci in a distributed setting. Just like the human retina, a distributed sensor network should focus on interesting events, while leaving sensing resources available to attend to emerging phenomena, analogous to foveate and peripheral vision in human vision systems. The system should be able to decide what to pay attention to and what to ignore, as there may be an overwhelming number of distractors.

To support such distributed attention, one must develop methods for selecting and shifting sensing foci, and algorithms for allocating sensing, processing, and communication resources among a set of potentially competing tasks. To deal with novel situations, a sensor network system should also be able to learn from data it observes over time, and to build models about normalcy and novelty for an environment, so that it can better anticipate or react to surprises in the future.

8.10 Smart meter

A smart meter is usually an electronic device that records consumption of electric energy in intervals of an hour or less and communicates that information at least daily back to the utility for monitoring and billing. Smart meters enable two-way communication between the meter and the central system. Unlike home energy monitors, smart meters can gather data for remote reporting. Such an advanced metering infrastructure (AMI) differs from traditional automatic meter reading (AMR) in that it enables two-way communications with the meter.

Overview

The term Smart Meter often refers to an electricity meter, but it also may mean a device measuring natural gas or water consumption. Similar meters, usually referred to as interval or time-of-use meters, have existed for years, but "Smart Meters" usually involve real-time or near real-time sensors, power outage notification, and power quality monitoring. These additional features are more than simple automated meter reading (AMR). They are similar in many respects to Advanced Metering Infrastructure (AMI) meters. Interval and time-of-use meters historically have been installed to measure commercial and industrial customers, but may not have automatic reading.

Research by Which?, the UK consumer group, showed that as many as one in three confuse smart meters with energy monitors, also known as in-home display monitors. The roll-out of smart meters is one strategy for energy savings. While energy suppliers in the UK could save around £300 million a year from their introduction, consumer benefits will depend on people actively changing their energy use. For example, time of use tariffs offering lower rates at off-peak times, and selling electricity back to the grid with net metering, may also benefit consumers.

The installed base of smart meters in Europe at the end of 2008 was about 39 million units, according to analyst firm Berg Insight. Globally, Pike Research found that smart meter shipments were 17.4 million units for the first quarter of 2011. Vision gain has determined that the value of the global smart meter market will reach $7bn in 2012. Smart meters may be part of a smart grid, but alone, they do not constitute a smart grid.

Brief history

In 1972, Theodore George “Ted” Paraskevakos, while working with Boeing in Huntsville, Alabama, developed a sensor monitoring system which used digital transmission for security, fire and medical alarm systems as well as meter reading capabilities for all utilities. This technology was a spin-off of the automatic telephone line identification system, now known as Caller ID. In 1974, Mr. Paraskevakos was awarded a U.S. patent for this technology. In 1977, he launched Metretek, Inc., which developed and produced the first fully automated, commercially available remote meter reading and load management system. Since this system was developed pre-Internet, Metretek utilized the IBM series mini-computer. For this approach, Mr. Paraskevakos and Metretek were awarded multiple patents.

Purpose

Since the inception of electricity deregulation and market-driven pricing throughout the world, utilities have been looking for a means to match consumption with generation. Traditional electrical and gas meters only measure total consumption, and so provide no information of when the energy was consumed at each metered site. Smart meters provide a way of measuring this site-specific information, allowing utility companies to introduce different prices for consumption based on the time of day and the season.

Utility companies propose that from a consumer perspective, smart metering offers potential benefits to householders. These include, a) an end to estimated bills, which are a major source of complaints for many customers b) a tool to help consumers better manage their energy purchases - stating that smart meters with a display outside their homes could provide up-to-date information on gas and electricity consumption and in doing so help people to manage their energy use and reduce their energy bills. Electricity pricing usually peaks at certain predictable times of the day and the season. In particular, if generation is constrained, prices can rise if power from other jurisdictions or more costly generation is brought online.

Proponents assert that billing customers at a higher rate for peak times will encourage consumers to adjust their consumption habits to be more responsive to market prices and assert further, that regulatory and market design agencies hope these "price signals" could delay the construction of additional generation or at least the purchase of energy from higher priced sources, thereby controlling the steady and rapid increase of electricity prices. There are some concerns, however, that low income and vulnerable consumers may not benefit from intraday time-of-use tariffs.

An academic study based on existing trials showed that homeowners' electricity consumption on average is reduced by approximately 3-5%. The ability to connect/disconnect service and read meter consumption remotely are major labor savings for the utility and can result in large layoffs of meter readers.

Technology

Of all smart meter technologies, one critical technological problem is communication. Each meter must be able to reliably and securely communicate the information collected to some central location. Considering the varying environments and locations where meters are found, that problem can be daunting. Among the solutions proposed are: the use of cell and pager networks, satellite, licensed radio, combination licensed and unlicensed radio, and power line communication. Not only the medium used for communication purposes, but also the type of network used, is critical. As such, one would find: fixed wireless, mesh network or a combination of the two. There are several other potential network configurations possible, including the use of Wi-Fi and other internet related networks. To date no one solution seems to be optimal for all applications. Rural utilities have very different communication problems from urban utilities or utilities located in difficult locations such as mountainous regions or areas ill-served by wireless and internet companies.

In addition to communication with the head-end network, smart meters may need to be part of a Home Area Network which can include an In-Premise Display and a hub to interface

one or more meters with the head end. Technologies for this network will vary from country to country but include Power line communication and ZigBee.

Protocols

ANSI C12.18 is an ANSI standard that describes a protocol used for two-way communications with a meter, mostly used in North American markets. The C12.18 standard is written specifically for meter communications via an ANSI Type 2 Optical Port, and specifies lower-level protocol details. ANSI C12.19 specifies the data tables that will be used. ANSI C12.21 is an extension of C12.18 written for modem instead of optical communications, so it is better suited to automatic meter reading.

IEC 61107 is a communication protocol for smart meters published by the IEC that is widely used for utility meters in the European Union. It is superseded by IEC 62056, but remains in wide use because it is simple and well-accepted. It sends ASCII data using a serial port. The physical media are either modulated light, sent with an LED and received with a photodiode, or a pair of wires, usually modulated by EIA-485. The protocol is half-duplex. IEC 61107 is related to, and sometimes wrongly confused with, the FLAG protocol. Ferranti and Landis+Gyr were early proponents of an interface standard that eventually became a sub-set of IEC1107.

The Open Smart Grid Protocol (OSGP) is a family of specifications published by the European Telecommunications Standards Institute (ETSI) used in conjunction with the ISO/IEC 14908 control networking standard for smart metering and smart grid applications. Millions of smart meters based on OSGP are deployed worldwide. Numerous major security flaws in the OSGP protocol have been identified. There is a growing trend toward the use of TCP/IP technology as a common communication platform for Smart Meter applications, so that utilities can deploy multiple communication systems, while using IP technology as a common management platform.

A universal metering interface would allow for development and mass production of smart meters and smart grid devices prior to the communication standards being set, and then for the relevant communication modules to be easily added or switched when they are. This would lower the risk of investing in the wrong standard as well as permit a single product to be used globally even if regional communication standards vary. Some smart meters may use a test IR LED to transmit non-encrypted usage data that bypasses meter security by transmitting lower level data in real time.

Data management

The other critical technology for Smart Meter systems is the information technology at the utility that integrates the Smart Meter networks with the utility applications, such as billing and CIS. This includes the Meter Data Management system. It also is important for Smart Grid implementations that power line communication (PLC) technologies used within the home over a Home Area Network (HAN), are standardized and compatible. The HAN allows HVAC systems and other household appliances to communicate with the smart meter, and from there to the utility.

Currently there are several broadband or narrowband standards in place, or being developed, that are not yet compatible. In order to address this issue, the National Institute for Standards and Technology (NIST) established the PAP15 group, which will study and recommend coexistence mechanisms with a focus on the harmonization of PLC standards for the HAN. The objective of the group is to ensure that all PLC technologies selected for the HAN will coexist as a minimum. The two main broadband PLC technologies selected are the HomePlug AV / IEEE 1901 and ITUT G.hn technologies. Technical working groups within these organizations are working to develop appropriate coexistence mechanisms. 	The HomePlug Powerline Alliance has developed a new standard for smart grid HAN communications called the HomePlug Green PHY specification. It is interoperable and coexistent with the widely deployed HomePlug AV technology and with the new IEEE 1901 global standard and is based on Broadband OFDM technology. ITU-T commissioned in 2010 a new project called G.hnem, to address

the home networking aspects of energy management, built upon existing Low Frequency Narrowband OFDM technologies.

Google.org's PowerMeter, until its demise in 2011, was able to use a smart meter for tracking electricity usage,[90] as can eMeter's Energy Engage as in, for example, the PowerCentsDC(TM) demand response program. Google Power Meter was retired in September 2011.

Security

Implementing security protocols that will protect these devices from malicious attacks and new exploits has been a problematic task. One proposed smart meter data verification method involves analyzing the data in real time to detect anomalies. By identifying exploits as they are being leveraged by attackers, this intrusion detection system (IDS) will mitigate the suppliers' risks of energy theft by consumers and denial-of-service attacks by hackers.

Figure 8.28 Examples of Smart metering applications

Advanced metering infrastructure

Advanced Metering Infrastructure (AMI) are systems that measure, collect, and analyze energy usage, and communicate with metering devices such as electricity meters, gas meters, heat meters, and water meters, either on request or on a schedule. These systems include hardware, software, communications, consumer energy displays and controllers, customer associated systems, Meter Data Management (MDM) software, and supplier business systems. Government agencies and utilities are turning toward advanced metering infrastructure (AMI) systems as part of larger “Smart Grid” initiatives. AMI extends current advanced meter reading (AMR) technology by providing two way meter communications, allowing commands to be sent toward the home for multiple purposes, including “time-of-use” pricing information, demand-response actions, or remote service disconnects.

8.11 Home automation for the elderly and disabled

The form of home automation called assistive domestics focuses on making it possible for older adults and people with disabilities to remain at home, safe and comfortable. Home automation is becoming a viable option for older adults and people with disabilities who would prefer to stay in the comfort of their homes rather than move to a healthcare facility. This field uses much of the same technology and equipment as home automation for security, entertainment, and energy conservation but tailors it towards older adults and people with disabilities.

Advantages

Home automation is being implemented into more and more homes of older adults and people with disabilities in order to maintain their independence and safety. These smart homes allow older adults and people with disabilities to stay in their homes where they feel comfortable, instead of moving to a costly health care facility. The transition to a health care facility can cause a lot of anxiety and home automation can either prevent or delay this anxiety (Cheek 2005). For the disabled smart homes give them opportunity for independence, which will help them gain confidence and determination. Smart homes can provide both older adults and people with disabilities with many different types of emergency assistance systems, security features, fall prevention, automated timers, and alerts. These systems allow for the individual to feel secure in their homes knowing that help is only minutes away. Smart home systems will make it possible for family members to monitor their loved ones from anywhere with an internet connection.

Systems

Home automation for healthcare can range from very simple alerts to lavish computer controlled network interfaces. Some of the monitoring or safety devices that can be installed in a home include lighting and motion sensors, environmental controls, video cameras, automated timers, emergency assistance systems, and alerts.

Security

In order to maintain the security of the home many home automation systems integrate features such as remote keyless entry systems which will allow seniors to view who is at the door and then remotely open the door. Home networks can also be programmed to automatically lock doors and shut blinds in order to maintain privacy.

Reminder systems

Home automation systems may include automatic reminder systems for the elderly (Cheek 2005). Such systems are connected to the Internet and make announcements over an intercom. They can prompt about doctor’s appointments and taking medicine, as well as everyday activities such as turning off the stove, closing the blinds, locking doors, etc. Users choose what activities to be reminded of. The system can be set up to automatically perform tasks based on user activity, such as turning on the lights or adjusting room temperature when the user enters specified areas.

Other systems can remind users at home or away from home to take their medicine, and how much, by using an alarm wristwatch with text message and medical alert. Reminder systems can also remind about everyday tasks such as eating lunch or walking the dog. Some communities offer free telephone reassurance services to residents, which includes both safety check calls as well as reminders. These services have been credited with saving the lives of many elderly and senior citizens who choose to remain at home.

Medication Dispensing and Spoon-feeding

Smart homes can implement medication dispensing devices in order to ensure that necessary medications are taken at appropriate times (Cheek 2005). Automated pill dispensers can dispense only the pills that are to be taken at that time and are locked; versions are available for Alzheimer’s patients that have a lock on them (Cheek 2005). For diabetic patients a talking glucose monitor allows the patient to check their blood sugar level and take the appropriate injection (Cheek 2005). Digital thermometers are able to recognize a fever and alert physicians. Blood pressure and pulse monitors dispense hypertensive medications when needed. There are also spoon-feeding robots.